LAT-TD-XXXXX   LAT FSW Qualification Test Procedure, CMDFNC_001

        24 of 28

	[image: image1.jpg]



	Document #
	Date effective

	
	LAT-TD-XXXX
	

	
	Author(s)
	Supersedes

	
	Shantha Condamoor
	

	
	Sergio Maldonado
	

	
	
	

	
	Subsystem/Office

	
	Electronics & DAQ Subsystem

	Document Title

	LAT FSW Qualification Test Procedure: 
CMDFNC_001: Soft Reset


Change History Log
	Revision
	Effective Date
	Description of Changes

	01
	XXXXX
	Original

	
	
	

	
	
	

	
	
	

	
	
	


Table of Contents
3Table of Contents


51.
Scope


51.1
Test Suite


51.2
Test ID


51.3
Requirement(s) Tested


62.
DEFINITIONS AND ACRONYMS


62.1
Definitions


62.2
Acronyms


73.
REFERENCES


73.1
Applicable Documents


84.
REQUIREMENTS


84.1
Test Data, Equipment and Software


84.2
Quality Assurance


84.3
Safety


94.4
Warnings, Cautions, and Notes


94.5
General Instructions


105.
SETUP


105.1
Hardware Setup


125.2
Software Setup


155.3
Setup Validation


166.
TEST PROCEDURE FOR CMDFNC_001


166.1
Test Objective


176.2
Test Input Files


176.3
Test Output Files


186.4
Test Preparation


196.5
Test Procedure


247.
TEST POST_CONDITIONS AND OVERALL OUTCOME


247.1
Test Post-Conditions


247.2
Overall Outcome of CMDFNC_001


268.
CERTIFICATION


27Appendix A: Deviations from THE Qualification Test Procedure





1. Scope

This document describes the procedure followed for qualification testing of the LAT FSW. This Qualification Test Procedure document describes one of the qualification tests executed to verify compliance with the requirements defined in the “Flight Software Specification – Level III” (LAT-SS-00399).
1.1 Test Suite
CMDFNC
The Test Suite is a logical grouping of a set of tests based on common functionalities and can be executed as a suite of tests in some order when the pre-conditions and post-conditions for each test within the suite have been met successfully. 

This test procedure CMDFNC_001 belongs in the Test Suite CMDFNC under the FST Project.

1.2 Test ID
CMDFNC_001
Test CMDFNC_001 contains one main script, CMDFNC_001, which can be run when the SIU or an EPU is in the Boot or Application Mode.
1.3 Requirement(s) Tested

The Qualification Test Procedure described herein is performed to verify that the FSW satisfies the following requirement(s), quoted from the Flight Software Specification – Level III:
	Requirement Number
	Requirement Name
	Requirement

	5.3.1.4.1
	Soft Reset
	SIU FSW shall perform a primary boot in response to an internal or external soft-reset command.


If the requirement(s) quoted above cite external documents (e.g., “…Further details are provided in [11]”), consult LAT-SS-00399 for the list of citations.
2. DEFINITIONS AND ACRONYMS
The following terms, abbreviations, and acronyms are used in this document:

2.1 Definitions
	Hz
	Hertz, unit of frequency 

	s, sec
	seconds

	V
	Volt

	W
	Watt

	s, sec
	seconds

	
	

	
	


2.2 Acronyms
	CAL
	Calorimeter

	EGSE
	Electrical Ground Support Equipment

	GASU
	Global trigger Anti-collision Spacecraft Unit

	PTR
	Post Test Review

	TEM
	Tower Electronics Module

	TKR
	Tracker

	TPS
	Tower Power Supply

	TRR
	Test Readiness Review

	QAE
	Quality Assurance Engineer

	TE
	Test Engineer

	
	

	
	

	
	

	
	

	
	


3. REFERENCES
The list below provides documents that are to be used as references for this procedure:
3.1 Applicable Documents

	Document Number
	Description

	SPECIFICATIONS
	

	LAT-SS-00399
	LAT Flight Software Level III Specification

	1196 EI-S46310-000
	GLAST 1553 Bus Protocol Interface Control

	LAT-TD-02659
	LAT Flight Software Telecommand and Telemetry Formats

	LAT-TD-0561
	The Virtual Spacecraft (VSC)

	
	

	PROCEDURES
	

	
	

	
	

	PLANS
	

	LAT-MD-00039
	Performance Assurance Implementation Plan

	LAT-MD-00078
	GLAST LAT System Safety Program Plan

	LAT-MD-00404
	LAT Contamination Control Plan

	LAT-MD-00408
	LAT Program Instrument Performance Verification Plan

	LAT-SS-00296
	T & DF Test Plan

	LAT-TD-00297
	LAT Electronics Test Plan

	LAT-TD-00786
	LAT Flight Software Test Plan

	
	

	
	

	DRAWINGS
	

	
	

	
	

	OTHER
	

	LAT-MD-00091
	GLAST Quality Manual

	LAT-MD-00471
	Control of Nonconforming Product

	LAT-MD-00472
	Corrective and Preventative Action

	
	

	
	

	
	


4. REQUIREMENTS

This section lists the requirements that shall be followed during the LAT FSW Qualification Testing process.

The Performance Assurance Implementation Plan, LAT-MD-00039, shall be utilized to ensure that the products produced by the GLAST LAT project intended for design qualification, flight and critical ground support equipment usage meet the required levels of quality and functionality for their intended purposes.

The LAT Program Instrument Performance Verification Plan, LAT-MD-00408, shall be utilized to address the testing to be performed at the unit/subsystem and instrument level for flight qualification, proto-flight and acceptance testing phases. Also included are the necessary processes/procedures and systems assurance activities.
4.1 Test Data, Equipment and Software

This procedure shall follow the requirements found in the Control of Nonconforming Product, LAT-MD-00471. This document establishes the method to identify and control nonconforming product developed by the LAT project team.

4.2 Quality Assurance

This procedure shall follow the requirements found in the Corrective and Preventative Action document, LAT-MD-00472 and the GLAST Quality Manual, LAT-MD-00091.

The Corrective and Preventative Action document establishes the method to be used to initiate, implement, evaluate and record corrective and preventive actions. The GLAST Quality Manual defines the methods implemented by the GLAST LAT project to ensure consistent quality of all processes for procurement, design, development and production of flight hardware, flight software and all associated ground support equipment interfacing with flight hardware and software.

4.3 Safety

This procedure shall follow the requirements found in the GLAST LAT System Safety Program Plan, LAT-MD-00078. This document defines all phases of the LAT program including: design, development, fabrication, handling, transportation, storage, test, assembly and operation.
WARNING: When high voltages are present extreme care should be exercised.

4.4 Warnings, Cautions, and Notes

The following SAFETY ALERTS are intended to create awareness of the potential safety hazards and the steps that must be taken to avoid accidents. These same alerts are used throughout this document to identify specific hazards that may endanger personnel and/or equipment.
Identification of every conceivable hazardous situation is impossible. Therefore, all personnel have the responsibility to diligently exercise safe practices whenever exposed to this equipment.
WARNING: Indicates a potential hazardous situation which, if not avoided, could result in death or injury.

CAUTION: Indicates a potential hazardous situation which, if not avoided, could result in damage to equipment.

Note: Indicates a notification of information that is important, but not hazard related.

4.5 General Instructions

This qualification test procedure shall be conducted on a formal basis to its latest approved and released version. The designated Software QAE shall be notified 24 hours prior to the start of this procedure. Software QAE may monitor the execution of all or part of this procedure should they elect to do so.
The Test Engineer conducting this test shall read this document in its entirety and resolve any apparent ambiguities before beginning the procedures described herein.

Deviations from the procedures described in this document and breaks in hardware or software configuration can only be initiated by the Test Engineer, must be approved by QA, and must be documented in Appendix A. 

Any nonconformance/defect/anomaly is to be reported in JIRA. Refer to the LAT Flight Software Test Plan LAT-TD-00786 for guidance. Do not alter or break configuration if a failure occurs. Notify Software Quality Assurance. 

All success conditions for a test must be met for the test to pass.
5. SETUP
This section describes the hardware and software configuration used for the qualification test described later in this document. Any break from configuration or deviation from a particular procedure must be authorized by the Quality Assurance Engineer and documented in Appendix A.

5.1 Hardware Setup

The list below indicates the equipment that is used to execute the tests described in this document.
	Hardware Unit
	Manufacturer
	GLAT Number
	Firmware Version (where applicable)

	VSC: “Virtual Spacecraft” – mv2304-based VME crate system acts as a space craft simulator and Bus Controller for the Remote Terminal (SIU) on the 1553 bus.
	SLAC

1553 Bus Connectivity to RT 3 (SIU)

Ethernet Bus Connectivity to Host, VSC. 

Power Supply
	lat-elf18
	

	SIU – RAD750 crate acts as the Remote Terminal 3 on the 1553 bus.
	cCPCI chassis

RAD750 Processor

SIB housing EEPROM and 1553 Interface hardware.

Ethernet Board

Serial Port connection
	lat-elf17
	

	EPU0 – RAD750 crate
	cCPCI chassis

RAD750 Processor

SIB housing EEPROM

Ethernet Board

Serial Port connection
	lat-elf24
	

	EPU1 – RAD750 crate
	cCPCI chassis

RAD750 Processor

SIB housing EEPROM

Ethernet Board

Serial Port connection
	lat-elf25
	

	1553 cables and couplers
	
	
	

	Unix or Linux Host

Establishes connection between VSC and the terminal from which the test is run.
	SLAC
	
	

	Power supply for SIU
	
	
	


The Figure below depicts the Testbed on which this qualification test is performed. All Testbed hardware is itemized by GLAT number and firmware version in the preceding table. The particular hardware units utilized in this qualification test are highlighted in the same table. [image: image2.png]From EBM

(GASU)
LVDS Science Heater Switch Environmental
Data Interface Monitor (LEDs) Simulator Boards
vsc
Host
1-PPS
(LTX proxy 1-PPS (GPS —
interface to 1
i or CPU clock) —
test scripts, (x2) PDU (x2)
test outputs) - -
1553 Environmental
mv2304 CPU|S 1553 J« swidhes {10 Registers
Ethernet [sumr 1553|
O Power Switches
Discrete pios| Mo [S[tce
| discretexs |
Ethernet Lines Board
Board *
GASU (x2)
Terminal VSC back-end VSC front-end
interface interface
I N
EPU (x3)
CRU
RAD750
SIB e '
< )
FES
Terminal
Iy
Ethernet
FES
TEM
Conmrs TEM (x16) TEM
(8 PC Boards)|
Front End Sim. Boards:
5 TKR: 1/twr = 16 boards
> (B cAL 1twr= 16 boaras
L,| fes |2 | AcD:2icomer =8 boards
Controller g
{0 Bcard) |ACD FES Board
1 Output to AEM
B | =
Clock Signal
ACD Controllers
(2 PC Boards) &@Ag —

n

0 VSC
LVDS Science
Interface




Figure 1. FSW Testbed
5.2 Software Setup

The software required to prepare for and execute the tests described in this Qualification Test Procedure document is itemized in this section.

5.2.1 Test Tools

The following table specifies the test executive used to run this qualification test, and identifies the other software tools used to support the execution of the test. The “Software Version Number” column identifies the version number of the test tool being used; alternatively, this column identifies the hardcopy attachment to this document that records the version of the tool being used (e.g., “Attachment 1”). The “Path to Attachment” column identifies the directory in which the electronic copy of any hardcopy attachment is saved (if applicable).
	Software
	Description of Software
	Software Version Number
(or Specify Attachment Number)
	Path to Attachment
(If Applicable)

	LTX
	LAT Test Executive
	
	

	FMX
	File Management Extra
	
	

	GLEAM 
	GLast Event Analysis Machine
	
	

	FES
	Front End Simulator system software
	
	

	VSC
	Virtual Spacecraft system software 
	
	

	VPI
	VSC Python/Proxy Interface 
	
	

	
	
	
	


5.2.2 Test Scripts
The following table identifies the test scripts that are run to execute this qualification test. The “Script Version Number” column identifies which version of the script is being used; alternatively, this column identifies the hardcopy attachment to this document that records the version of the script being used (e.g., “Attachment 1”). The “Path to Attachment” column identifies the directory in which the electronic copy of any hardcopy attachment is saved (if applicable).
	Test Script
	Description of Test Script
	Script Version Number
(or Specify Attachment Number)
	Path to Attachment
(If Applicable)

	primary_boot_test.py
	This script must be run after the RAD750 crate has undergone Primary Boot (COLD, COMMNDED etc.) but not Secondary Boot. This script verifies the pre-conditions for some tests that need to be conducted in the PBC mode only. 

This test does not alter the state of the CPU. It leaves the CPU in Boot mode.
	
	

	secondary_boot_test.py
	This script must be run after the RAD750 crate has undergone Secondary Boot. This script verifies  the pre-conditions for some tests that need to be conducted in the SBC mode where

· The RTOS has been loaded,

· All FSW Application Software Packages have been loaded,

· All tasks initialized and started.

This test does not alter the state of the CPU. It leaves the CPU in Application mode.
	
	

	version_test.py
	This script must be run after primary_boot_test or secondary_boot_test to verify the version numbers of the currently loaded FSW packages.
	
	

	primary_boot.py
	This script must be run if the pre-condition for a test necessitates that the RAD750 crate must be in the PBC mode. This script performs a hardware reset on the CPU through the Reset line of the VSC. It waits till the Primary Boot is completed and the Boot Housekeeping Telemetry starts coming in.
	
	

	secondary_boot.py
	This script must be run after the primary_boot_test to advance the RAD750 crate to Secondary Boot. This script meets the pre-conditions for some tests that need to be conducted in the Application Mode. In this mode, the CPU is ready to accept telecommands issued from the S/C that can be processed only in the Application Mode. It may also start sending Housekeeping and other telemetry.
	
	

	CMDFNC.cfg
	This is a configuration script for the CMDFNC suite. It contains information such as Current Mode (Boot or Application), LATUNITS used by this test (SIU, EPU0, EPU1) etc. and other parameters that are altered during the running of various scripts.
	
	

	CMDFNC_001.py
	This script can be run in the Boot or Application Modes. It is tested on SIU, EPU0, and EPU1.
	
	


5.2.3 Flight Software

This qualification test is performed on a complete, integrated Candidate Release of FSW. All FSW libraries under test are final Flight Unit Candidate versions. The test described in this document is designed to evaluate the particular FSW packages and constituents listed in the following table; for clarity, only those constituents that are the focus of the test are listed below. 
	FSW Package
	Constituent(s)

	ITC
	???

	
	

	
	

	
	


The Candidate Release into which these FSW constituents are integrated is identified in the following table. The “Candidate Release Build Number” column specifies the build number of the FSW Candidate Release under test (e.g., “B0-0-1”). The “Candidate Release Attachment Number” column identifies which hardcopy attachment to this document confirms the name and version number of the full set of FSW packages and constituents used in the test (e.g., “Attachment 2”). The “Path to Attachment” column specifies where in the test repository an electronic copy of the hardcopy attachment has been saved. 
	Candidate Release Build Number
	Candidate Release Attachment Number
	Path to Attachment

	
	
	


5.3 Setup Validation

5.3.1 Hardware Validation

The following signatures confirm that the Test Engineer and Quality Assurance Engineer have verified the GLAT numbers, firmware version numbers, and the proper connection of all hardware listed in the table in Section 5.1 (and shown in Figure 1).
__________________   
_____________ ________________   ______________
Date

Time

Test Engineer

QAE

5.3.2 Software Validation
The Test Engineer performs the following procedure to validate the software setup for this qualification test and records completion of the setup validation steps in the space provided.

	Step No.
	Description of Step
	Step Outcome

	1
	Record the version numbers of all test tools used to perform this qualification test in the table in Section 5.2.1
OR
Obtain version numbers of the test tools used to perform this qualification test by running the xxxx script. Pick up and label the hardcopy report generated by the xxxx script and attach it to this test procedure document. Record (1) the label for the attachment in table and (2) the directory location to which an electronic copy of the report was saved in Section 5.2.1.
	Complete/
Not Complete

	2
	Record the version numbers of all test scripts used to perform this qualification test in the table in Section 5.2.2
OR 

Obtain version numbers of the tesc scripts used to perform this qualification test by running the xxxx script. Pick up and label the hardcopy report generated by the xxxx script and attach it to this test procedure document. Record (1) the label for the attachment and (2) the directory location to which an electronic copy of the report was saved in Section 5.2.2.
	Complete/
Not Complete


The following signatures confirm that, using the procedure described in the previous table, the Test Engineer and Quality Assurance Engineer have verified that all versions of test support software, test scripts, and FSW constituents match those identified in Section 5.2.

__________________   
_____________ ________________   ______________
Date

Time

Test Engineer

QAE

6. TEST PROCEDURE FOR CMDFNC_001 
6.1 Test Objective

The following are the objectives of this test when tested in the Boot or Application Modes.

This test verifies that the SIU performs a primary boot in response to an internal or external soft-reset. 
This Test Objective is broken down into the following Test Sub-Objectives.

	Number
	Test Sub-Objective

	1
	Verify all operations on SIU only.

	2
	Verify all operations in both the Boot and Application Modes unless specified otherwise.

	3
	Verify all operations in both the Primary and Redundant SIUs unless specified otherwise.

	4
	In the Boot and Application modes, verifies external reset by sending the LBTRESET (Warm Reboot) command and noticing that the Boot Housekeeping Telemetry indicates that the COMMANDED type of reboot occurred in the BOOTYPE field.

	5
	In the Boot and Application modes, verifies internal reset with the Watchdog type of Reset. MEMLOADMEM command to the address 0xBF880058 with value 0 initiates this type of reset. Resultant Boot Housekeeping Telemetry is checked to verify that the WATCHDOG type of reboot occurred in the BOOTYPE field.

	6
	In the Boot mode, verifies internal reset with the Exception type of Reset. MEMLOADREG command to the Register 0x8C with value 0xFFFFFFFF initiates this type of reset. Resultant Boot Housekeeping Telemetry is checked to verify that the EXCEPTION type of reboot occurred in the BOOTYPE field.


As part of 1553 Interface and Command Validation tests described in CMDFNC_003, the following are also listed as Test Objectives and verified in this test.

	Number
	Test Sub-Objective

	1
	Verify that the supported PBC Telecommands and Telemetry exchanged across the 1553 Interface conform to  CCSDS packet structure:

	2
	Verify that the supported valid PBC Telecommands issued by S/C with LAT as their destination are received correctly and processed. 


6.2  Test Input Files

The following table identifies all auxiliary files (e.g., Front End Simulator data files, GLEAM data files) used as inputs to this qualification test. Note that not all qualification tests use input data of this type. The “Input File Version Number” column identifies the version number of the auxiliary file being used; alternatively, this column identifies the hardcopy attachment to this document that records the version of the file being used (e.g., “Attachment 1”). The “Path to Attachment” column specifies where in the test repository an electronic copy of the hardcopy attachment has been saved .
	Input File
	Description of Input File
	Input File Version Number (or Specify Attachment Number)
	Path to Attachment
(If Applicable)

	N/A
	N/A
	N/A
	N/A


6.3 Test Output Files

The following table identifies the log files, data dump files, and other output files generated by the test scripts executed during this qualification test. The “Output File Attachment” column identifies which attachment to this document is the hardcopy record of this script output data. The “Path to Attachment” column specifies where in the test repository an electronic copy of the hardcopy attachment has been saved.
	Output File
	Description of Output File
	Output File Attachment
	Path to Attachment

	ltx_host_xxx.log
	Host logfile
	
	

	VPI_apid_xxx.dat
	Telemetry output file
	
	

	VPI_mne_xxx.csv
	Telemetry mnemonic output file
	
	


6.4 Test Preparation

After the hardware and software setup has been validated, steps may be required to place the hardware and FSW in an operational mode in which the qualification test can be performed or otherwise complete preparations for the test to begin.

The Test Engineer carries out the following procedure to prepare for qualification tested and records completion of the test preparation steps in the space provided.
	Step No.
	Description of Step
	Step Outcome

	1
	Execute a Primary Boot of the SIU. The Primary Boot process takes about 2 minutes.
	Complete/
Not Complete

	2
	Run the primary_boot_test script to confirm that the SIU FSW is operating in Boot mode.
	Complete/
Not Complete

	3
	The primary_boot_test outputs a file named “PBC.log”. Retrieve the output file from the printer, label it, and attach it to this test procedure. See the table in Section 6.3 for additional details.
	Complete/
Not Complete

	4a
	If results of the primary_boot_test indicate that the SIU FSW is not operating in Boot mode, run the script primary_boot to reset the SIU to the Boot mode.
	Complete/
Not Complete

OR

Not Applicable

	4b
	If step 4a was performed, run the primary_boot_test script again to confirm the SIU is in Boot mode, then attach the resulting “PBC.log” file to this test procedure. See the table in Section 6.3 for additional details.
	Complete/
Not Complete

OR

Not Applicable


The Test Engineer and Quality Assurance Engineer verify that all test preparation steps are complete.
__________________   
_____________ ________________   ______________
Date

Time

Test Engineer

QAE

6.5 Test Procedure

A summary of the steps involved in this test procedure is provided below. It is primarily intended as a description of how the Test Objectives are verified in this test and how data is analyzed to qualify if each of the test objectives have been met.
The following MEM telecommands and MEM telemetry are used in this test.
The following PBC and MEM telecommands and PBC telemetry are used in this test.

PBC Telecommands

	APID
	FC
	Cmd. Packet (L)
	Description (L)

	0x640
	0
	LBTSTART
	Boot code no-op

	0x640
	1
	LBTRESET
	Warm reboot

	0x640
	2
	LBTERRDUMP
	Error code pop


MEM Telecommands
	0x644
	4
	LMEMLOADMEM
	Memory Write

	0x644
	6
	LMEMLOADREG
	Processor Register Write


PBC Telemetry

	APID
	Tlm. Packet (L)
	Description (L)

	0x200
	LBTHKP
	Boot housekeeping telemetry

	0x261
	LBTEPU0HKP
	EPU 0 Boot Housekeeping Telemetry

	0x262
	LBTEPU1HKP
	EPU 1 Boot Housekeeping Telemetry

	0x263
	LBTEPU2HKP
	EPU 2 Boot Housekeeping Telemetry


	Step No.
	Description of Step
	Step Completed?
(Yes/No)

	1
	Boot Mode Commanded Reset
Verify that Boot Housekeeping Telemetry is coming in. Save the Hi scrub addresses, BOOTPKTCOUNT, BOOTPKTACCEPT and other fields in the Telemetry.

Send the external reset command LBTRESET (Warm Reboot) command and notice that the Boot Housekeeping Telemetry indicates that the COMMANDED type of reboot occurred in the BOOTYPE field. Also the Hi scrub addresses and other error fields in this Telemetry are initialized to fresh values.
	

	2
	Boot Mode Watchdog Internal Reset

Verify that Boot Housekeeping Telemetry is coming in. Save the Hi scrub addresses, BOOTPKTCOUNT, BOOTPKTACCEPT and other fields in the Telemetry.

Indirectly verify internal reset with the Watchdog Reset. Send MEMLOADMEM command to the address 0xBF880058 with value 0 which initiates this type of reset. Resultant Boot Housekeeping Telemetry is checked to verify that the WATCHDOG type of reboot occurred in the BOOTYPE field. Also the Hi scrub addresses and other error fields in this Telemetry are initialized to fresh values.
	

	3
	Boot Mode Exception Internal Reset

Verify that Boot Housekeeping Telemetry is coming in. Save the Hi scrub addresses, BOOTPKTCOUNT, BOOTPKTACCEPT and other fields in the Telemetry.

Indirectly verify internal reset with the Exception Reset. Send MEMLOADREG command to the Register 0x8C with value 0xFFFFFFFF which initiates this type of reset. Resultant Boot Housekeeping Telemetry is checked to verify that the EXCEPTION type of reboot occurred in the BOOTYPE field. Also the Hi scrub addresses and other error fields in this Telemetry are initialized to fresh values.
	

	4
	Application Mode Commanded Reset
Verify that LAT Housekeeping Telemetry is coming in to ensure that SIU is in Application Mode. 

Send the external reset command LBTRESET (Warm Reboot) command and notice that the Boot Housekeeping Telemetry starts to come in after some time. Indicates that the COMMANDED type of reboot occurred in the BOOTYPE field. Also the Hi scrub addresses and other error fields in this Telemetry are initialized to fresh values.
	

	5
	Application Mode Watchdog Internal Reset

Verify that LAT Housekeeping Telemetry is coming in to ensure that SIU is in Application Mode. 

Indirectly verify internal reset with the Watchdog Reset. Send MEMLOADMEM command to the address 0xBF880058 with value 0 which initiates this type of reset. Resultant Boot Housekeeping Telemetry is checked to verify that the WATCHDOG type of reboot occurred in the BOOTYPE field. Also the Hi scrub addresses and other error fields in this Telemetry are initialized to fresh values.
	

	
	PBC 1553 interface and Command Verification Tests
The following tests verify the 1553 Interface and Command Validation aspects of PBC.
	

	
	Boot Mode PBC command Data Format and Integrity Errors

Verify that the  supported PBC Telecommands exchanged across the 1553 Interface conform to  CCSDS packet structure:

Verification that PBC Telecommands with valid Headers/Checksum are accepted and processed by FSW in the Boot Mode is performed here.
In the Boot mode, verifies by sending PBC commands with valid parameters and receiving the LBTHKP, which is the SIU Boot Housekeeping Telemetry for SIU. BOOTCOMMAND and Error fields in the Boot Housekeeping Telemetry are used to verify the Success or failure of execution of commands.
MEM telecommands is verified for CCSDS conformance in MEMMGT_001 and not in this test.
Boot Housekeeping telemetry is verified for CCSDS conformance in CMDFNC_002 and not in this test.
	

	
	Boot Mode PBC Command Format and Parameter Validation

 Verify that the supported PBC Telecommands issued by S/C with LAT as their destination are received correctly and processed. 

Verification that PBC Telecommands with valid Parameters are accepted and processed by FSW is performed in this test.
In the Boot mode, verifies by sending PBC commands with valid parameters and receiving the LBTHKP, which is the SIU Boot Housekeeping Telemetry for SIU. BOOTCOMMAND and Error fields in the Boot Housekeeping Telemetry are used to verify the Success or failure of execution of commands.
MEM telecommands is verified for Parameter Validation in MEMMGT_001 and not in this test.
Boot Housekeeping telemetry is verified for valid parameters in CMDFNC_002 and not in this test.
	


After the test setup has been validated, and test preparation steps are complete, the Test Engineer proceeds with the qualification test procedure itself, as described below, and records the outcome of each step during test execution. The outcome of each step is either “Complete” or “Not Complete” (for steps which involve no analysis or verification); or, “Pass” or “Fail” (for steps involved in verifying completion of test objectives and sub-objectives). 
	Step No.
	Description of Step
	Step Outcome

	1
	While RAD750 crates are in Boot mode, run the script CMDFNC_001 with the following command:
$ ltx run <CMDFNC_001 
	Complete/
Not Complete

	2
	The CMDFNC_001 script generates and xxx log file and sends the file to the printer. Retrieve the output file, label it, and attach it to this test procedure document. See the table in Section 6.3 for additional details.
	Complete/
Not Complete

	3
	[[Review the xxx output file and verify that the following entry appears followed by “PASS” or “FAIL”
Test Sub-Objective 

Boot Mode Commanded Reset
Boot Mode Watchdog Internal Reset
Boot Mode Exception Internal Reset
PBC 1553 interface and Command Verification Tests
Boot Mode PBC command Data Format and Integrity Errors

Boot Mode PBC Command Format and Parameter Validation

CMDFNC_001(1): Passed

This test verifies completion of all test sub-objectives in the Boot Mode.
	Pass/
Fail

	4
	Advance the RAD750 crates to Application Mode by running the script secondary_boot.
	Complete/
Not Complete

	5
	Run the secondary_boot_test script to confirm that the SIU and EPUs have undergone Secondary Boot and are operating in Application mode. This script generates an output file named “PBC.log”. Retrieve the output file from the printer, label it, and attach it to this test procedure. See the table in Section 6.3 for additional details.
	Complete/
Not Complete

	6a
	If results of the secondary_boot_test script indicate that the crates are not in Application mode, then run the secondary_boot script to advance the CPU to the Application mode. 
	Complete/
Not Complete

OR

Not Applicable

	6b
	If step 5a was performed, re-rerun the secondary_boot_test to ensure that crates are in Application mode. This script generates an output file named “PBC.log”. Retrieve the output file from the printer, label it, and attach it to this test procedure. See the table in Section 6.3 for additional details.
	Complete/
Not Complete

OR

Not Applicable

	7
	While FSW is in Application Mode, run the script CMDFNC_001 under LTX through the VSC.
	Complete/
Not Complete

	4
	[[Review the xxx output file and verify that the following entry appears followed by “PASS” or “FAIL”
Test Sub-Objective 

Application Mode Commanded Reset
Application Mode Watchdog Internal Reset
CMDFNC_001(2): Passed

This test verifies completion of all test sub-objectives in the Application Mode
	Pass/
Fail


Initial to confirm.
__________________   
_____________ ________________   ______________
Date

Time

Test Engineer

QAE

7. TEST POST_CONDITIONS AND OVERALL OUTCOME
7.1 Test Post-Conditions

The following post-conditions are analyzed and verified by the test script: 
	No.
	Post-Condition
	Post-Condition Met?
(Yes/No)

	1
	SIU in Boot Mode
	

	2
	
	

	3
	
	


The Test Engineer and Quality Assurance Engineer verify that all test post-conditions are met.

__________________   
_____________ ________________   ______________
Date

Time

Test Engineer

QAE

7.2 Overall Outcome of CMDFNC_001
Based on the analysis of the test results, the overall outcome of Test CMDFNC_001 is as follows:

□ Passed - all of the expected outcomes for the test were confirmed
□ Failed - one or more of the test outcomes were not confirmed
__________________  
________________   ______________

Date

    Test Engineer

QAE

8. CERTIFICATION
I certify that the information obtained under this test procedure is as represented and the information recorded in this document is complete and correct. Any deviations from test procedures described herein are identified in Appendix A.
_________________

__________________________
________________________
Date



Test Engineer (Print Name)

Test Engineer (Signature)
I certify that the information obtained through execution of this test procedure is as represented and the information recorded in this document is complete and correct. Execution of the test, storage of the results, and verification of outcomes were carried out in accordance with quality standards defined in the GLAST Quality Manual (LAT-MD-00091).
_________________
___________________________
_____________________________
Date


Software QA Engineer (Print Name)
Software QA Engineer (Sign)

I certify that the information obtained under this test procedure is as represented and the information recorded in this document is complete and correct. The test procedure, as designed and executed, does indeed verify that the FSW functionality under test satisfies the corresponding requirements from the Flight Software Specification – Level III.
_________________

________________________
__________________________
Date



FSW Manager (Print Name)

FSW Manager (Signature)
Appendix A: Deviations from THE Qualification Test Procedure
This section details any deviations from the hardware configuration, software configuration, or test procedure followed during the execution of the test or tests described in this Qualification Test Procedure document. All deviations from the approved procedure are agreed to by the Test Engineer and the Software Quality Engineer during the test execution session. All deviations must be reported during the Post Qualification Test Review, where their impact on the test results will be evaluated.

Hardware Deviations

Describe any deviations from the hardware configuration defined in Section 5.1. Name the hardware that was modified and describe the modifications. If hardware is replaced during execution of the test, name the replaced hardware, the manufacturer, and list an identification number (e.g., GLAT ID number).

______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Software Deviations
Describe any changes made to the software configuration under test or the software configuration used to support test execution, as defined in Section 5.2. Give version numbers of all FSW packages and test packages that were modified. Describe how the contents of the modified software load were verified. Describe these deviations for each test that was modified.
______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Procedural Deviations
Specify any deviations from the test procedure for the test being executed. If this document contains more than one test procedure, list the procedure by number (e.g., “CMDFNC_001”). List by number the steps modified or skipped. Provide a numbered sequence listing any added steps. Describe these deviations for each test that was modified.
______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________



































































































































































































































































































































































































































































Hard copies of this document are for REFERENCE ONLY and should not be considered the latest revision.

Hard copies of this document are for REFERENCE ONLY and should not be considered the latest revision. 


