	[image: image1.jpg]



	Document #
	Date effective

	
	LAT-TD-07114-03
	07/23/07

	
	Author(s)
	Supersedes

	
	Igor Volobouev
	LAT-TD-07114-02

	
	Shantha Condamoor
	

	
	Mark Arndt
	

	
	Subsystem/Office

	
	Electronics & DAQ Subsystem

	Document Title

	LAT FSW Qualification Test Procedure: 

EVTFIL_003: Event Filtering – Event Filter Reprogramming


Change History Log

	Revision
	Effective Date
	Description of Changes

	1
	08/15/05
	Original

	2
	01/04/06
	Updates for FSW B0-6-1 interfaces

	3
	07/23/07
	Updated the “Test Scripts” table in Section 5.2.2 to account for the fact that FES runs are now started using shell scripts.

Updated the “Test Input Files” table in Section 6.2  to list the full set of VPI configuration files. 
Revised step 2 in Section 6.5 to include commands to configure FSW to NOT autoboot from the lower bank of SIU. Also, added command to configure the test environment to point to the QSEP tools.
Updated the text of the final terminal output in the “Test Post-Conditions” table in Section 7.

	
	
	

	
	
	


Table of Contents

3Table of Contents


51.
Scope


51.1
Test Suite


51.2
Test ID


51.3
Requirement(s) Tested


72.
DEFINITIONS AND ACRONYMS


72.1
Definitions


72.2
Acronyms


83.
REFERENCES


83.1
Applicable Documents


94.
REQUIREMENTS


94.1
Test Data, Equipment and Software


94.2
Quality Assurance


104.3
Safety


104.4
Warnings, Cautions, and Notes


104.5
General Instructions


125.
SETUP


125.1
Hardware Setup


135.2
Software Setup


155.3
Setup Validation


166.
TEST PROCEDURE FOR EVTFIL_003


166.1
Test Objective


176.2
Test Input Files


186.3
Test Output Files


196.4
Test Preparation


206.5
Test Procedure


227.
TEST POST-CONDITIONS AND OVERALL OUTCOME


227.1
Test Post-Conditions


237.2
Overall Outcome of EVTFIL_003


248.
CERTIFICATION


25Appendix A: Deviations from THE Qualification Test Procedure


258.1
Hardware Deviations


258.2
Software Deviations


268.3
Procedural Deviations





1. Scope

This document describes the procedure followed for qualification testing of the LAT FSW. This Qualification Test Procedure document describes one of the qualification tests executed to verify compliance with the requirements defined in the “Flight Software Specification – Level III” (LAT-SS-00399).

1.1 Test Suite

EVTFIL

The Test Suite is a logical grouping of a set of tests based on common functionalities and can be executed as a suite of tests in some order when the pre-conditions and post-conditions for each test within the suite have been met successfully. 

This test procedure EVTFIL_003 belongs in the Test Suite EVTFIL under the FST Project. The EVTFIL tests verify the functionality of the LAT onboard event filter software.

1.2 Test ID

EVTFIL_003

Test EVTFIL_003 contains one main test script, evtfil_003.py, which can be run when the SIU and EPUs are operating in Boot Mode.

1.3 Requirement(s) Tested

The Qualification Test Procedure described herein is performed to verify that the FSW satisfies the following requirement(s), quoted from the Flight Software Specification – Level III:

	Requirement Number
	Requirement Name
	Requirement
	Level of Requirements Verification in This Test

	5.3.9.2
	Event Filter Reprogramming
	The event filtering software shall be reprogrammable via the SIU.
	Full

	5.3.9.4
	Event Filter Changes
	[Derived]

Changes to the configuration of the event filtering algorithms, initiated by command or autonomously, shall be indicated in telemetry.
	Partial (initiated by command tested here, autonomous changes tested in GRBPRC test suite)


If the requirement(s) quoted above cite external documents (e.g., “…Further details are provided in [11]”), consult LAT-SS-00399 for the list of citations.

2. DEFINITIONS AND ACRONYMS

The following terms, abbreviations, and acronyms are used in this document:

2.1 Definitions

	Hz
	Hertz, unit of frequency 

	s, sec
	seconds

	V
	Volt

	W
	Watt

	s, sec
	seconds

	
	

	
	


2.2 Acronyms

	CAL
	Calorimeter

	EGSE
	Electrical Ground Support Equipment

	FOV
	Field of View

	GASU
	Global trigger Anti-collision Spacecraft Unit

	GBM
	Gamma-ray Burst Monitor

	GRB
	Gamma Ray Bust

	PTR
	Post Test Review

	TEM
	Tower Electronics Module

	TKR
	Tracker

	TPS
	Tower Power Supply

	TRR
	Test Readiness Review

	QAE
	Quality Assurance Engineer

	TE
	Test Engineer

	
	

	
	

	
	

	
	

	
	


3. REFERENCES

The list below provides documents that are to be used as references for this procedure:

3.1 Applicable Documents

	Document Number
	Description

	SPECIFICATIONS
	

	LAT-SS-00399
	LAT Flight Software Level III Specification

	LAT-TD-02659
	LAT Flight Software Telecommand and Telemetry Formats

	LAT-TD-0561
	The Virtual Spacecraft (VSC)

	
	

	PROCEDURES
	

	
	

	
	

	PLANS
	

	LAT-MD-00039
	Performance Assurance Implementation Plan

	LAT-MD-00078
	GLAST LAT System Safety Program Plan

	LAT-MD-00404
	LAT Contamination Control Plan

	LAT-MD-00408
	LAT Program Instrument Performance Verification Plan

	LAT-SS-00296
	T & DF Test Plan

	LAT-TD-00297
	LAT Electronics Test Plan

	LAT-TD-00786

LAT-MD-00104
	LAT Flight Software Test Plan

LAT Flight Software Management Plan

	
	

	
	

	DRAWINGS
	

	
	

	
	

	OTHER
	

	LAT-MD-00091
	GLAST Quality Manual

	LAT-MD-00471
	Control of Nonconforming Product

	LAT-MD-00472
	Corrective and Preventative Action

	
	

	
	

	
	


4. REQUIREMENTS

This section lists the requirements that shall be followed during the LAT FSW Qualification Testing process.

The Performance Assurance Implementation Plan, LAT-MD-00039, shall be utilized to ensure that the products produced by the GLAST LAT project intended for design qualification, flight and critical ground support equipment usage meet the required levels of quality and functionality for their intended purposes.

The LAT Program Instrument Performance Verification Plan, LAT-MD-00408, shall be utilized to address the testing to be performed at the unit/subsystem and instrument level for flight qualification, proto-flight and acceptance testing phases. Also included are the necessary processes/procedures and systems assurance activities.

4.1 Test Data, Equipment and Software

This procedure shall follow the requirements found in the Control of Nonconforming Product, LAT-MD-00471. This document establishes the method to identify and control nonconforming product developed by the LAT project team.

4.2 Quality Assurance

This procedure shall follow the requirements found in the Corrective and Preventative Action document, LAT-MD-00472 and the GLAST Quality Manual, LAT-MD-00091.

The Corrective and Preventative Action document establishes the method to be used to initiate, implement, evaluate and record corrective and preventive actions. The GLAST Quality Manual defines the methods implemented by the GLAST LAT project to ensure consistent quality of all processes for procurement, design, development and production of flight hardware, flight software and all associated ground support equipment interfacing with flight hardware and software.

4.3 Safety

This procedure shall follow the requirements found in the GLAST LAT System Safety Program Plan, LAT-MD-00078. This document defines all phases of the LAT program including: design, development, fabrication, handling, transportation, storage, test, assembly and operation.

WARNING: When high voltages are present extreme care should be exercised.

4.4 Warnings, Cautions, and Notes

The following SAFETY ALERTS are intended to create awareness of the potential safety hazards and the steps that must be taken to avoid accidents. These same alerts are used throughout this document to identify specific hazards that may endanger personnel and/or equipment.

Identification of every conceivable hazardous situation is impossible. Therefore, all personnel have the responsibility to diligently exercise safe practices whenever exposed to this equipment.

WARNING: Indicates a potential hazardous situation which, if not avoided, could result in death or injury.

CAUTION: Indicates a potential hazardous situation which, if not avoided, could result in damage to equipment.

Note: Indicates a notification of information that is important, but not hazard related.

4.5 General Instructions

This qualification test procedure shall be conducted on a formal basis to its latest approved and released version. The designated Software QAE shall be notified 24 hours prior to the start of this procedure. Software QAE may monitor the execution of all or part of this procedure should they elect to do so.

The Test Engineer conducting this test shall read this document in its entirety and resolve any apparent ambiguities before beginning the procedures described herein.

Deviations from the procedures described in this document and breaks in hardware or software configuration can only be initiated by the Test Engineer, must be approved by QA, and must be documented in Appendix A. 

Any nonconformance/defect/anomaly is to be reported in JIRA. Refer to the LAT Flight Software Test Plan LAT-TD-00786 for guidance. Do not alter or break configuration if a failure occurs. Notify Software Quality Assurance. 

All success conditions for a test must be met for the test to pass.

5. SETUP

This section describes the hardware and software configuration used for the qualification test described later in this document. Any break from configuration or deviation from a particular procedure must be authorized by the Quality Assurance Engineer and documented in Appendix A.

5.1 Hardware Setup

The list below indicates the equipment that is used to execute the tests described in this document.

	Hardware Unit
	Manufacturer
	GLAT Number
	Firmware Version (where applicable)

	Front End Simulator (FES): 1
	SLAC
	
	

	Virtual Spacecraft (VSC): 1
	SLAC
	
	

	Spacecraft Interface Unit (SIU): 1
	SLAC
	
	

	Event Processing Unit (EPU): 2
	SLAC
	
	

	Tower Electronics Module (TEM): 16
	SLAC
	
	

	Global Trigger AEM Signal distribution unit (GASU): 1
	SLAC
	
	

	Power Distribution Unit (PDU):1
	SLAC
	
	

	Unix or Linux host

Establishes connection between VSC and the terminal from which the test is run.
	
	
	


The Figure below depicts the Testbed on which this qualification test is performed. The particular hardware units utilized in this test are itemized by GLAT number and firmware version in the preceding table.

[image: image2.png]From EBM

(GASU)
LVDS Science Heater Switch Environmental
Data Interface Monitor (LEDs) Simulator Boards
vsc
Host
1-PPS
(LTX proxy 1-PPS (GPS —
interface to 1
i or CPU clock) —
test scripts, (x2) PDU (x2)
test outputs) - -
1553 Environmental
mv2304 CPU|S 1553 J« swidhes {10 Registers
Ethernet [sumr 1553|
O Power Switches
Discrete pios| Mo [S[tce
| discretexs |
Ethernet Lines Board
Board *
GASU (x2)
Terminal VSC back-end VSC front-end
interface interface
I N
EPU (x3)
CRU
RAD750
SIB e '
< )
FES
Terminal
Iy
Ethernet
FES
TEM
Conmrs TEM (x16) TEM
(8 PC Boards)|
Front End Sim. Boards:
5 TKR: 1/twr = 16 boards
> (B cAL 1twr= 16 boaras
L,| fes |2 | AcD:2icomer =8 boards
Controller g
{0 Bcard) |ACD FES Board
1 Output to AEM
B | =
Clock Signal
ACD Controllers
(2 PC Boards) &@Ag —

n

0 VSC
LVDS Science
Interface




Figure 1. FSW Testbed

5.2 Software Setup

The software required to prepare for and execute the tests described in this Qualification Test Procedure document is itemized in this section.

5.2.1 Test Tools

The following table specifies the test executive used to run this qualification test, and identifies the other software tools used to support the execution of the test. The “Software Version Number” column identifies the version number of the test tool being used; alternatively, this column identifies the hardcopy attachment to this document that records the version of the tool being used (e.g., “Attachment 1”). The “Path to Attachment” column identifies the directory in which the electronic copy of any hardcopy attachment is saved (if applicable).

	Software
	Description of Software
	Software Version Number
(or Specify Attachment Number)
	Path to Attachment
(If Applicable)

	LTX
	LAT Test Executive
	
	

	LCAT
	Telecommand creation and telemetry decoding software
	
	

	FMX
	File Management Extra
	
	

	VPI
	VSC Proxy Interface 
	
	

	VSCP
	Python API for Virtual Spacecraft software
	
	

	STU
	Standard Test Utilities
	
	


5.2.2 Test Scripts

The following table identifies the test scripts that are run to execute this qualification test. The “Script Version Number” column identifies which version of the script is being used; alternatively, this column identifies the hardcopy attachment to this document that records the version of the script being used (e.g., “Attachment 1”). The “Path to Attachment” column identifies the directory in which the electronic copy of any hardcopy attachment is saved (if applicable).

	Test Script
	Description of Test Script
	Script Version Number
(or Specify Attachment Number)
	Path to Attachment
(If Applicable)

	evtfil_003.py


	Main LTX qualification test execution script for EVTFIL_003
	
	

	EVTFIL_003.xml


	LTX test description file for EVTFIL_003
	
	

	evtfil_003_analyze.py


	Test data/results analysis script for EVTFIL_003
	
	

	cfg-fes.sh
	FES configuration shell script to initialize and start simulation run from a Host Computer.
	
	


5.2.3 Flight Software

This qualification test is performed on a complete, integrated Candidate Release of FSW. All FSW libraries under test are final Flight Unit Candidate versions. The test described in this document is designed to evaluate the particular FSW packages and constituents listed in the following table; for clarity, only those constituents that are the focus of the test are listed below. 

	FSW Package
	Constituent(s)

	LPA
	

	EFC
	

	GFC_DB
	


The Candidate Release into which these FSW constituents are integrated is identified in the following table. The “Candidate Release Build Number” column specifies the build number of the FSW Candidate Release under test (e.g., “B0-0-1”). The “Candidate Release Attachment Number” column identifies which hardcopy attachment to this document confirms the name and version number of the full set of FSW packages and constituents used in the test (e.g., “Attachment 2”). The “Path to Attachment” column specifies where in the test repository an electronic copy of the hardcopy attachment has been saved. 

	Candidate Release Build Number
	Candidate Release Attachment Number
	Path to Attachment

	
	
	


5.3 Setup Validation

5.3.1 Hardware Validation

The following signatures confirm that the Test Engineer and Quality Assurance Engineer have verified the GLAT numbers, firmware version numbers, and the proper connection of all hardware listed in the table in Section 5.1.

__________________   
_____________ ________________   ______________
Date

Time

Test Engineer

QAE


5.3.2 Software Validation

The Test Engineer performs the following procedure to validate the software setup for this qualification test and records completion of the setup validation steps in the space provided.

	Step No.
	Description of Step
	Step Outcome


	1
	Record the version numbers of all test tools used to perform this qualification test in the table in Section 5.2.1.
	Complete/

Not Complete

	2
	Record the version numbers of all test scripts used to perform this qualification test in the table in Section 5.2.2.
	Complete/

Not Complete

	3
	Record the version numbers of the FSW constituents and the Candidate Release on which this test is performed in Section 5.2.3.
	Complete/

Not Complete


The following signatures confirm that, using the procedure described in the previous table, the Test Engineer and Quality Assurance Engineer have verified that all versions of test support software, test scripts, and FSW constituents match those identified in Section 5.2.

__________________   
_____________ ________________   ______________
Date

Time

Test Engineer

QAE


6. TEST PROCEDURE FOR EVTFIL_003

6.1 Test Objective

This test verifies that:

· The filter code can be replaced by another code by means of uploading a different code file.

· The filter configuration can be changed by means of uploading a different configuration file.

· Changes in the filter configuration initiated by telecommands are reflected in the telemetry stream.

The requirement that autonomous changes in the filter configuration are reflected in the telemetry stream could not be verified at the time of this writing.

This Test Objective is broken down into the following Test Sub-Objective(s).

	Number
	Test Sub-Objective

	1
	Checks that the set of events accepted by the filter is reproducible in two testbed runs with identical configuration. This verifies that the change in the set of accepted events actually indicates a change in the filter.

	2
	Perform another run and issue a TOO telecommand in the middle of the run. Search the telemetry stream for the message which indicates change in the filter configuration caused by TOO command.

	3
	Rerun the simulation with an altered filter configuration files and verify that alternative configuration produces variations in the the set of passed events.

	4
	Rerun the simulation with an altered filter code (add a different event handler ) and verify that the the set of passed events is as expected.


6.2 Test Input Files

The following table identifies all auxiliary files (e.g., Front End Simulator data files, GLEAM data files) used as inputs to this qualification test. Note that not all qualification tests use input data of this type. The “Input File Version Number” column identifies the version number of the auxiliary file being used; alternatively, this column identifies the hardcopy attachment to this document that records the version of the file being used (e.g., “Attachment 1”). The “Path to Attachment” column specifies where in the test repository an electronic copy of the hardcopy attachment has been saved .

	Input File
	Description of Input File
	Input File Version Number (or Specify Attachmnt Number)
	Path to Attachment
(If Applicable)

	orbit_15min.txt
	Simulated orbit file
	
	

	hibg6min_v7r3p4
	FES file simulating cosmic ray background data
	
	

	libpres.o
	Alternative filter code file
	
	

	libgfc_db_fswtest0.o
	Alternative filter configuration file
	
	

	liblpa_db_data.o
	Alternative FSW configuration file with modified LPA settings
	
	

	latc/latc_daq.xml
	LATC configuration file
	
	

	latc/latc_ignore.lat
	LATC ignore map
	
	

	cfg/lat_hkp_mne.txt
	 VPI input file to configure archiving of LAT housekeeping specific mnemonics
	 
	 

	cfg/lat_diag_mne.txt
	 VPI input file to configure archiving of LAT diagnostic specific mnemonics
	 
	 

	cfg/lat_hkp_apid.txt
	 VPI input file to configure archiving of LAT housekeeping specific apids
	 
	 

	cfg/lat_diag_apid.txt
	 VPI input file to configure archiving of LAT diagnostic specific apids
	 
	 

	cfg/lat_sci_apid.txt
	 VPI input file to configure archiving of LAT science specific apids
	 
	 

	cfg/vsc_cmd_apid.txt
	 VPI input file to configure archiving of VSC command specific apids
	 
	 


6.3 Test Output Files

The following table identifies the log files, data dump files, and other output files generated by the test scripts executed during this qualification test. The “Output File Attachment Number” column identifies which attachment to this document is the hardcopy record of this script output data. The “Path to Attachment” column specifies where in the test repository an electronic copy of the hardcopy attachment has been saved.

	Output File
	Description of Output File
	Output File Attachment Number
	Path to Attachment

	bg_default_0.lsf
	Science data telemetry, LSF format file, for default filter run1
	
	

	bg_default_0.txt
	Text file which contains FES and VSC configuration information for the above run
	
	

	bg_default_1.lsf
	Science data telemetry, LSF format file, for default filter run 2
	
	

	bg_default_1.txt
	Text file which contains FES and VSC configuration information for the above run
	
	

	bg_with_too.lsf
	Science data telemetry, LSF format file, for filter run with TOO.
	
	

	bg_with_too.txt
	Text file which contains FES and VSC configuration information for the above run
	
	

	prescale_filter.lsf
	Science data telemetry, LSF format file, for alternative filter code
	
	

	prescale_filter.txt
	Text file which contains FES and VSC configuration information for the above run
	
	

	bg_prescale_15.lsf
	Science data telemetry, LSF format file, for alternative filter configuration
	
	

	bg_prescale_15.txt
	Text file which contains FES and VSC configuration information for the above run
	
	


6.4 Test Preparation

After the hardware and software setup has been validated, steps may be required to place the hardware and FSW in an operational mode in which the qualification test can be performed or otherwise complete preparations for the test to begin.

The Test Engineer carries out the following procedure to prepare for qualification tested and records completion of the test preparation steps in the space provided.

	Step No.
	Description of Step
	Step Outcome

	1
	Confirm that the VSC is powered up:

-- The VSC GPS module should display time since last reset.
	Complete/
Not Complete

	2
	Confirm that the Front End Simulator is powered up:

-- The power LED on all FES computers (lat-ent1, lat-ent2, ..., lat-ent11) should be on.

-- The voltage indicator on the Xantrex power supply marked "FES-FEED" should display 48 ± 0.5V, and the current should be above 5A.

-- The state indicators on all FES electronic boards should be flashing.
	Complete/

Not Complete

	3
	Confirm that the SIU and the GASU are powered up:

-- The voltage indicator on the SIU/GASU Xantrex power supply should display 28 ± 0.2V.

-- The SIU feed "POWER ON" switch on the regulated feeds bus protection unit (BPU) should be flipped "ON". The SIU voltage indicator on the BPU should display 28 ± 0.2V. The SIU current as shown by the BPU indicator should be above 0.4A

-- The DAQ feed "POWER ON" switch on the BPU should be flipped "ON". The DAQ voltage indicator on the BPU should display 28 ± 0.2V.
	Complete/

Not Complete


The Test Engineer and Quality Assurance Engineer verify that all test preparation steps are complete.

__________________   
_____________ ________________   ______________
Date

Time

Test Engineer

QAE


6.5 Test Procedure

After the test setup has been validated, and test preparation steps are complete, the Test Engineer proceeds with the qualification test procedure itself, as described below, and records the outcome of each step during test execution. The outcome of each step is either “Complete” or “Not Complete” (for steps which involve no analysis or verification); or, “Pass” or “Fail” (for steps involved in verifying completion of test objectives and sub-objectives).

	Step No.
	Description of Step
	Step Outcome


	1
	Verify presence of the simulated science data on the FES nodes. On a Linux host, run the command

fesdata show hibg6min_v7r3p4

It should display

Directory :

 1955625085  23-Dec-2005 10:32:48  ffffffffff  hibg6min_v7r3p4
	Complete / 
Not Complete

	2
	Set up the FSW to NOT autoboot from the lower bank of SIU; set environment to make QSEP available:

unsetenv LTX_C_AUTOBOOT

setenv VPI_AUTOBOOT_LO 0xc000

setenv VPI_AUTOBOOT_HI 0x5400

setenv QSEP_F_PROD $GFSROOT
	Complete /
Not Complete

	3
	Launch the data acquisition script. The expected run time for this script is 45 min:

ltx run EVTFIL_003 --ips=lat-hobbit2,lat-elf21,lat-ent1,lat-elf23,lat-elf24,lat-elf25 --prot=ethernet,xyplex,xyplex,xyplex,xyplex,xyplex

The data acquisition script performs five testbed runs using the same set of Monte Carlo events and generates  LSF files listed in the table in Section 6.3. The following sequence of operations is performed for each testbed run:

1) A set of spacecraft timestamped coordinates and attitudes is scheduled for simulation on the VSC using the VSC interface M7 (“magic seven”) class. The simulated orbit file used for “magic seven” telecommand scheduling is the same as the orbit file used to produce the background simulation. The orbit file format is described at

http://confluence.slac.stanford.edu/display/DC2/Orbit+and+Pointing+History
2) The physics run is configured using two or more LPACONFIGURE telecommands.

3) The LPASTART telecommand is sent to the spacecraft. This changes the LAT Instrument Manager operation mode from QUIESCENT to PHYSICS and initiates data taking by the LAT. 

4) The LIM “State” telemetry packets are received. The LPA task state, as identified by the LIMTLPASTATE field, is required to be “Running”. The LIM operation mode, as identified by the LIMTOPMODE field, is required to be PHYSICS. The "CmdConfirm" command validation telemetry packet with successful status is expected after the LIM “State” packet with the right properties. The timeout for “CmdConfirm” telemetry reception after the LPASTART telecommand is set to 30 seconds.

5) The FES run is started by talking to the FES system via the LTX interface. FES timing is periodic for all runs with 100 Hz frequency.

6) After a waiting period, the FES run is stopped. The length of the waiting period is set to 3 min for each run.

7) The LPASTOP telecommand is sent to the spacecraft.

8) The "CmdConfirm" command validation telemetry packet is expected. After receiving “CmdConfirm”, one or more  LIM “State” telemetry packets is received, until the LIMTLPASTATE field of the LIM “State” packet indicates that that the state of the LPA task is “Idle” and the LIMTOPMODE field indicates that the instrument operation mode is QUIESCENT. The timeout for reception of LIM “State” telemetry with right properties is set to 30 seconds after the LPASTOP telecommand.

During each run all packets received by the proxy on the VSC science stream are written to an LSF file on a hard disk. 

Note that changes in the filter code and configuration require system reboot.

Upon successful completion of the data acquisition stage, the script displays

DAQ script finished YYYY-MM-DD HH:MM:SS.UUUUUU
	Complete /
Not Complete

	4
	The data acquisition script automatically runs the data analysis script when all testbed runs are complete.

The data analysis script extracts the set of event numbers from the LSF files generated in the four testbed runs.

These sets are required to be identical for the first two runs (up to a small difference due to run stop time jitter). The sets obtained in the third and fourth run are required to be different from each other and from the set obtained in the first run.
	Complete/
Not Complete



	5
	Review the outputs of the data analysis script on the test terminal. The test passes if every line of the output contains the word “PASS”. Successful completion of this step fulfills all test objectives and sub-objectives.

Example data analysis script output is shown below:

Filtering algorithm is reproducible : PASS

TOO filter reconfiguration is indicated in the telemetry : PASS

Change in the filter code is confirmed in simulated data taking : PASS

Change in the filter configuration is confirmed in simulated data taking : PASS

Combined test status : PASS
	Pass/

Fail

	6
	Press “Enter” in the terminal to end the test
	Complete/
Not Complete


Initial to confirm.

__________________   
_____________ ________________   ______________
Date

Time

Test Engineer

QAE


7. TEST POST-CONDITIONS AND OVERALL OUTCOME

7.1 Test Post-Conditions

The following post-conditions are analyzed and verified by the test script: 

	No.
	Post-Condition
	Post-Condition Met?
(Yes/No)

	1
	Upon successful completion of the data acquisition stage, LAT flight software is in BOOT mode

Terminal displays

LAT LPBCBOOTSTAT 2 PBC_SW_MODE_COMMAND

SIU LPBCBOOTTYPE 5 COMMAND
	


The Test Engineer and Quality Assurance Engineer verify that all test post-conditions are met.

__________________   
_____________ ________________   ______________
Date

Time

Test Engineer

QAE


7.2 Overall Outcome of EVTFIL_003

Based on the analysis of the test results, the overall outcome of Test EVTFIL_003 is as follows:

□ Passed - all of the expected outcomes for the test were confirmed

□ Failed - one or more of the test outcomes were not confirmed

__________________  
________________   ______________

Date

    Test Engineer

QAE


8. CERTIFICATION

I certify that the information obtained under this test procedure is as represented and the information recorded in this document is complete and correct. Any deviations from test procedures described herein are identified in Appendix A.

_________________

__________________________
________________________

Date



Test Engineer (Print Name)

Test Engineer (Signature)

I certify that the information obtained through execution of this test procedure is as represented and the information recorded in this document is complete and correct. Execution of the test, storage of the results, and verification of outcomes were carried out in accordance with quality standards defined in the GLAST Quality Manual (LAT-MD-00091).

_________________
___________________________
_____________________________

Date


Software QA Engineer (Print Name)
Software QA Engineer (Sign)


I certify that the information obtained under this test procedure is as represented and the information recorded in this document is complete and correct. The test procedure, as designed and executed, does indeed verify that the FSW functionality under test satisfies the corresponding requirements from the Flight Software Specification – Level III.

_________________

________________________
__________________________

Date



FSW Manager (Print Name)

FSW Manager (Signature)

Appendix A: Deviations from THE Qualification Test Procedure

This section details any deviations from the hardware configuration, software configuration, or test procedure followed during the execution of the test or tests described in this Qualification Test Procedure document. All deviations from the approved procedure are agreed to by the Test Engineer and the Software Quality Engineer during the test execution session. All deviations must be reported during the Post Qualification Test Review, where their impact on the test results will be evaluated.

Hardware Deviations

Describe any deviations from the hardware configuration defined in Section 5.1. Name the hardware that was modified and describe the modifications. If hardware is replaced during execution of the test, name the replaced hardware, the manufacturer, and list an identification number (e.g., GLAT ID number).

________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Software Deviations

Describe any changes made to the software configuration under test or the software configuration used to support test execution, as defined in Section 5.2. Give version numbers of all FSW packages and test packages that were modified. Describe how the contents of the modified software load were verified. Describe these deviations for each test that was modified.

________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Procedural Deviations

Specify any deviations from the test procedure for the test being executed. If this document contains more than one test procedure, list the procedure by number (e.g., “MEMMGT_001”). List by number the steps modified or skipped. Provide a numbered sequence listing any added steps. Describe these deviations for each test that was modified.

________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Hard copies of this document are for REFERENCE ONLY and should not be considered the latest revision.


