	[image: image1.jpg]



	Document #
	Date effective

	
	LAT-TD-07154-03
	07/23/07

	
	Author(s)
	Supersedes

	
	Sergio Maldonado
	LAT-TD-07154-02

	
	Shantha Condamoor
	

	
	Mark Arndt
	

	
	Subsystem/Office

	
	Electronics & DAQ Subsystem

	Document Title

	LAT FSW Qualification Test Procedure: 

FECALB_003: Charge Injection Calibration – TKR Trigger Check


Change History Log

	Revision
	Effective Date
	Description of Changes

	01
	08/17/05
	Original

	02
	02/14/06
	Updates for FSW B-0602 interfaces

	3
	07/23/07
	Updated the “Test Scripts” table in Section 5.2.2 to account for the fact that FES runs are now started using UNIX shell scripts as opposed to directly from lat-ent1 through xyplex commands.

Updated the “Test Input Files” table in Section 6.2  to list the full set of VPI configuration files. 

Updated the text of the final terminal output in the “Test Post-Conditions” table in Section 7.

Added LCAT to the Test Tools table.

Revised step 1 in Section 6.5 to include commands to configure FSW to autoboot from the lower bank of SIU. Also, updated the lat-hobbit machine name to lat-hobbit2.

	
	
	

	
	
	


Table of Contents

3Table of Contents


51.
Scope


51.1
Test Suite


51.2
Test ID


61.3
Requirement(s) Tested


72.
DEFINITIONS AND ACRONYMS


72.1
Definitions


72.2
Acronyms


83.
REFERENCES


83.1
Applicable Documents


94.
REQUIREMENTS


94.1
Test Data, Equipment and Software


94.2
Quality Assurance


104.3
Safety


104.4
Warnings, Cautions, and Notes


104.5
General Instructions


125.
SETUP


125.1
Hardware Setup


135.2
Software Setup


155.3
Setup Validation


166.
TEST PROCEDURE FOR FECALB_003


166.1
Test Objective


176.2
Test Input Files


186.3
Test Output Files


186.4
Test Preparation


196.5
Test Procedure


216.6
Test Analysis


217.
TEST POST-CONDITIONS AND OVERALL OUTCOME


217.1
Test Post-Conditions


227.2
Overall Outcome of FECALB_003


238.
CERTIFICATION


24Appendix A: Deviations from THE Qualification Test Procedure


248.1
Hardware Deviations


248.2
Software Deviations


258.3
Procedural Deviations





1.  Scope

This document describes the procedure followed for qualification testing of the LAT FSW. This Qualification Test Procedure document describes one of the qualification tests executed to verify compliance with the requirements defined in the “Flight Software Specification – Level III” (LAT-SS-00399).

1.1  Test Suite

FECALB

This suite contains tests for the LAT charge injection calibration software.

1.1.1  Relationship of FECALB_003 to Other Test Suites 

Responsibility for verifying 1553 interface and telecommand validation and processing requirements is shared between the FECALB test suite, OPMODE test suite, and the CMDFNC test suite. In particular, the FECALB and OPMODE tests verify that the subset of telecommands and telemetry associated with the charge injection calibration (LCI) FSW have the correct format, can be received over the 1553 interface and processed, and that telecommands with valid parameters are correctly processed. By contrast, the CMDFNC suite tests these telecommands and telemetry when packet structures and parameters are invalid. See the CMDFNC test procedure documents for a complete discussion of how responsibility for testing the entire telecommand and telemetry interface is mapped among various qualification test suites. 

The following LCI telecommands are used in test FECALB_003.

	LCI  Commands

	
	APID
	FC
	
	Cmd. Packet (I)
	Description (I)
	

	
	0x67C
	1
	
	LCICALIBRATE
	Initiate the calibration
	

	


1.2  Test ID

FECALB_003

Test FECALB_003 targets configuration, acquisition and delivery of TKR large injection charge DAC measurements by the charge injection calibration software.

1.3  Requirement(s) Tested

The Qualification Test Procedure described herein is performed to verify that the FSW satisfies the following requirement(s), quoted from the Flight Software Specification – Level III:

	Requirement Number
	Requirement Name
	Requirement

	5.3.12.3
	TKR Trigger Check
	FSW shall collect such data as is necessary to determine the per channel trigger efficiency for large injection charge (DAC 63).


If the requirement(s) quoted above cite external documents (e.g., “…Further details are provided in [11]”), consult LAT-SS-00399 for the list of citations.

2.  DEFINITIONS AND ACRONYMS

The following terms, abbreviations, and acronyms are used in this document:

2.1  Definitions

	Hz
	Hertz, unit of frequency 

	s, sec
	seconds

	V
	Volt

	W
	Watt

	s, sec
	seconds

	
	

	
	


2.2  Acronyms

	CAL
	Calorimeter

	EGSE
	Electrical Ground Support Equipment

	FOV
	Field of View

	GASU
	Global trigger Anti-collision Spacecraft Unit

	GBM
	Gamma-ray Burst Monitor

	GRB
	Gamma Ray Bust

	PTR
	Post Test Review

	TEM
	Tower Electronics Module

	TKR
	Tracker

	TPS
	Tower Power Supply

	TRR
	Test Readiness Review

	QAE
	Quality Assurance Engineer

	TE
	Test Engineer

	
	

	
	

	
	

	
	

	
	


3.  REFERENCES

The list below provides documents that are to be used as references for this procedure:

3.1  Applicable Documents

	Document Number
	Description

	SPECIFICATIONS
	

	LAT-SS-00399
	LAT Flight Software Level III Specification

	1196 EI-S46310-000
	GLAST 1553 Bus Protocol Interface Control

	LAT-TD-02659
	LAT Flight Software Telecommand and Telemetry Formats

	LAT-TD-0561
	The Virtual Spacecraft (VSC)

	LAT-SS-00010-1

LAT-TD-00169
	GLAST LAT Performance Specification - Level II(b) Specification

GTFE Specification

	
	

	PROCEDURES
	

	
	

	
	

	PLANS
	

	LAT-MD-00039
	Performance Assurance Implementation Plan

	LAT-MD-00078
	GLAST LAT System Safety Program Plan

	LAT-MD-00404
	LAT Contamination Control Plan

	LAT-MD-00408
	LAT Program Instrument Performance Verification Plan

	LAT-SS-00296
	T & DF Test Plan

	LAT-TD-00297
	LAT Electronics Test Plan

	LAT-TD-00786

LAT-MD-00104
	LAT Flight Software Test Plan

LAT Flight Software Management Plan

	
	

	
	

	DRAWINGS
	

	
	

	
	

	OTHER
	

	LAT-MD-00091
	GLAST Quality Manual

	LAT-MD-00471
	Control of Nonconforming Product

	LAT-MD-00472
	Corrective and Preventative Action

	
	

	
	

	
	


4.  REQUIREMENTS

This section lists the requirements that shall be followed during the LAT FSW Qualification Testing process.

The Performance Assurance Implementation Plan, LAT-MD-00039, shall be utilized to ensure that the products produced by the GLAST LAT project intended for design qualification, flight and critical ground support equipment usage meet the required levels of quality and functionality for their intended purposes.

The LAT Program Instrument Performance Verification Plan, LAT-MD-00408, shall be utilized to address the testing to be performed at the unit/subsystem and instrument level for flight qualification, proto-flight and acceptance testing phases. Also included are the necessary processes/procedures and systems assurance activities.

4.1  Test Data, Equipment and Software

This procedure shall follow the requirements found in the Control of Nonconforming Product, LAT-MD-00471. This document establishes the method to identify and control nonconforming product developed by the LAT project team.

4.2  Quality Assurance

This procedure shall follow the requirements found in the Corrective and Preventative Action document, LAT-MD-00472 and the GLAST Quality Manual, LAT-MD-00091.

The Corrective and Preventative Action document establishes the method to be used to initiate, implement, evaluate and record corrective and preventive actions. The GLAST Quality Manual defines the methods implemented by the GLAST LAT project to ensure consistent quality of all processes for procurement, design, development and production of flight hardware, flight software and all associated ground support equipment interfacing with flight hardware and software.

4.3  Safety

This procedure shall follow the requirements found in the GLAST LAT System Safety Program Plan, LAT-MD-00078. This document defines all phases of the LAT program including: design, development, fabrication, handling, transportation, storage, test, assembly and operation.

WARNING: When high voltages are present extreme care should be exercised.

4.4  Warnings, Cautions, and Notes

The following SAFETY ALERTS are intended to create awareness of the potential safety hazards and the steps that must be taken to avoid accidents. These same alerts are used throughout this document to identify specific hazards that may endanger personnel and/or equipment.

Identification of every conceivable hazardous situation is impossible. Therefore, all personnel have the responsibility to diligently exercise safe practices whenever exposed to this equipment.

WARNING: Indicates a potential hazardous situation which, if not avoided, could result in death or injury.

CAUTION: Indicates a potential hazardous situation which, if not avoided, could result in damage to equipment.

Note: Indicates a notification of information that is important, but not hazard related.

4.5  General Instructions

This qualification test procedure shall be conducted on a formal basis to its latest approved and released version. The designated Software QAE shall be notified 24 hours prior to the start of this procedure. Software QAE may monitor the execution of all or part of this procedure should they elect to do so.

The Test Engineer conducting this test shall read this document in its entirety and resolve any apparent ambiguities before beginning the procedures described herein.

Deviations from the procedures described in this document and breaks in hardware or software configuration can only be initiated by the Test Engineer, must be approved by QA, and must be documented in Appendix A. 

Any nonconformance/defect/anomaly is to be reported in JIRA. Refer to the LAT Flight Software Test Plan LAT-TD-00786 for guidance. Do not alter or break configuration if a failure occurs. Notify Software Quality Assurance. 

All success conditions for a test must be met for the test to pass.

5.  SETUP

This section describes the hardware and software configuration used for the qualification test described later in this document. Any break from configuration or deviation from a particular procedure must be authorized by the Quality Assurance Engineer and documented in Appendix A.

5.1  Hardware Setup

The list below indicates the equipment that is used to execute the tests described in this document.

	Hardware Unit
	Manufacturer
	GLAT Number
	Firmware Version (where applicable)

	Front End Simulator (FES): 1
	SLAC
	
	

	Virtual Spacecraft (VSC): 1
	SLAC
	
	

	Spacecraft Interface Unit (SIU): 1
	SLAC
	
	

	Tower Electronics Module (TEM): 16
	SLAC
	
	

	Global Trigger AEM Signal distribution unit (GASU): 1
	SLAC
	
	

	Power Distribution Unit (PDU):1
	SLAC
	
	

	Unix or Linux host

Establishes connection between VSC and the terminal from which the test is run.
	
	
	


The Figure below depicts the Testbed on which this qualification test is performed. The particular hardware units utilized in this test are itemized by GLAT number and firmware version in the preceding table.

[image: image2.png]From EBM

(GASU)
LVDS Science Heater Switch Environmental
Data Interface Monitor (LEDs) Simulator Boards
vsc
Host
1-PPS
(LTX proxy 1-PPS (GPS —
interface to 1
i or CPU clock) —
test scripts, (x2) PDU (x2)
test outputs) - -
1553 Environmental
mv2304 CPU|S 1553 J« swidhes {10 Registers
Ethernet [sumr 1553|
O Power Switches
Discrete pios| Mo [S[tce
| discretexs |
Ethernet Lines Board
Board *
GASU (x2)
Terminal VSC back-end VSC front-end
interface interface
I N
EPU (x3)
CRU
RAD750
SIB e '
< )
FES
Terminal
Iy
Ethernet
FES
TEM
Conmrs TEM (x16) TEM
(8 PC Boards)|
Front End Sim. Boards:
5 TKR: 1/twr = 16 boards
> (B cAL 1twr= 16 boaras
L,| fes |2 | AcD:2icomer =8 boards
Controller g
{0 Bcard) |ACD FES Board
1 Output to AEM
B | =
Clock Signal
ACD Controllers
(2 PC Boards) &@Ag —

n

0 VSC
LVDS Science
Interface




Figure 1. FSW Testbed

5.2  Software Setup

The software required to prepare for and execute the tests described in this Qualification Test Procedure document is itemized in this section.

5.2.1  Test Tools

The following table specifies the test executive used to run this qualification test, and identifies the other software tools used to support the execution of the test. The “Software Version Number” column identifies the version number of the test tool being used; alternatively, this column identifies the hardcopy attachment to this document that records the version of the tool being used (e.g., “Attachment 1”). The “Path to Attachment” column identifies the directory in which the electronic copy of any hardcopy attachment is saved (if applicable).

	Software
	Description of Software
	Software Version Number
(or Specify Attachment Number)
	Path to Attachment
(If Applicable)

	LTX
	LAT Test Executive
	
	

	LCAT
	LAT Command  & Telemetry DB tool in Python
	
	

	FES
	Front End Simulator system software
	
	

	VSC
	Virtual Spacecraft system software 
	
	

	VSCP
	VSC for Python
	
	

	VPI
	VSC Proxy Interface 
	
	

	DFI
	Dataflow Interface
	
	

	DFIP
	Dataflow Interface for Python
	
	


5.2.2  Test Scripts

The following table identifies the test scripts that are run to execute this qualification test. The “Script Version Number” column identifies which version of the script is being used; alternatively, this column identifies the hardcopy attachment to this document that records the version of the script being used (e.g., “Attachment 1”). The “Path to Attachment” column identifies the directory in which the electronic copy of any hardcopy attachment is saved (if applicable).

	Test Script
	Description of Test Script
	Script Version Number
(or Specify Attachment Number)
	Path to Attachment
(If Applicable)

	FECALB_003.py


	Main LTX qualification test execution script for FECALB_003
	
	

	FECALB_003_analyze.py


	Test data/results analysis script for FECALB_003
	
	

	LDF_dumper.py
	LDF data dumper, customized for FECALB 
	
	

	LDF_iterator.py
	 LDF data iterator, customized for FECALB
	
	

	init_siu.vx
	SIU FSW load script
	
	

	cfg-fes.sh
	FES configuration to initialize and start calibration simulaton run
	
	


5.2.3  Flight Software

This qualification test is performed on a complete, integrated Candidate Release of FSW. All FSW libraries under test are final Flight Unit Candidate versions. The test described in this document is designed to evaluate the particular FSW packages and constituents listed in the following table; for clarity, only those constituents that are the focus of the test are listed below. 

	FSW Package
	Constituent(s)

	LCI
	lci

	LSEC
	lsec


The Candidate Release into which these FSW constituents are integrated is identified in the following table. The “Candidate Release Build Number” column specifies the build number of the FSW Candidate Release under test (e.g., “B0-0-1”). The “Candidate Release Attachment Number” column identifies which hardcopy attachment to this document confirms the name and version number of the full set of FSW packages and constituents used in the test (e.g., “Attachment 2”). The “Path to Attachment” column specifies where in the test repository an electronic copy of the hardcopy attachment has been saved. 

	Candidate Release Build Number
	Candidate Release Attachment Number
	Path to Attachment

	
	
	


5.3  Setup Validation

5.3.1  Hardware Validation

The following signatures confirm that the Test Engineer and Quality Assurance Engineer have verified the GLAT numbers, firmware version numbers, and the proper connection of all hardware listed in the table in Section 5.1.

__________________   
_____________ ________________   ______________
Date

Time

Test Engineer

QAE


5.3.2  Software Validation

The Test Engineer performs the following procedure to validate the software setup for this qualification test and records completion of the setup validation steps in the space provided.

	Step No.
	Description of Step
	Step Outcome


	1
	Record the version numbers of all test tools used to perform this qualification test in the table in Section 5.2.1
	Complete/

Not Complete

	2
	Record the version numbers of all test scripts used to perform this qualification test in the table in Section 5.2.2
	Complete/

Not Complete

	3
	Record the version numbers of the FSW constituents and the Candidate Release on which this test is performed in Section 5.2.3
	Complete/

Not Complete

	4
	Record the version numbers of all input files used to perform this qualification test in the table in Section 6.2
	Complete/

Not Complete


The following signatures confirm that, using the procedure described in the previous table, the Test Engineer and Quality Assurance Engineer have verified that all versions of test support software, test scripts, input files, and FSW constituents match those identified in Sections 5.2 and 6.2.

__________________   
_____________ ________________   ______________
Date

Time

Test Engineer

QAE


6.  TEST PROCEDURE FOR FECALB_003

6.1  Test Objective

This test verifies that:

· The FSW is capable of configuring front-end electronics and collecting data needed for TKR trigger efficiency calibration per channel for a DAC 63 injection value.

Trigger efficiency can be determined by analyzing the TKR strip hits present in the event data.

Note that charge injection software executes solely on the SIU.

The FES does not contain a mechanism that fully supports simulation of charge injection calibration activities.  The resultant event data will not contain values consistent with the flight hardware.  However, the acquisition, format, and delivery of such events are identical.  Since all test objectives focus solely on these mechanisms, and do not involve analysis of raw data values, this approach is deemed acceptable for requirement verification purposes.

This Test Objective is broken down into the following Test Sub-Objective(s).

	Number
	Test Sub-Objective

	1
	Verify TKR event contribution present in datagrams for fixed DAC 63 charge injection runs


The test script assumes that all changes in the hardware configuration are reported in the data stream (per FSW requirement 5.3.17.4).

6.2  Test Input Files

The following table identifies all auxiliary files (e.g., Front End Simulator data files, GLEAM data files) used as inputs to this qualification test. Note that not all qualification tests use input data of this type. The “Input File Version Number” column identifies the version number of the auxiliary file being used; alternatively, this column identifies the hardcopy attachment to this document that records the version of the file being used (e.g., “Attachment 1”). The “Path to Attachment” column specifies where in the test repository an electronic copy of the hardcopy attachment has been saved .

Unless specifically mentioned in the table, nominal number of DAC settings, readout samples, and nominal layer readout split are used.

	Input File
	Description of Input File
	Input File Version Number (or Specify Attachmnt Number)
	Path to Attachment
(If Applicable)

	FECALB_003.xml
	LTX test configuration file
	
	

	cfg/lat_hkp_mne.txt
	 VPI input file to configure archiving of LAT housekeeping specific mnemonics
	 
	

	cfg/lat_diag_mne.txt
	 VPI input file to configure archiving of LAT diagnostic specific mnemonics
	 
	

	cfg/lat_hkp_apid.txt
	 VPI input file to configure archiving of LAT housekeeping specific apids
	 
	

	cfg/lat_diag_apid.txt
	 VPI input file to configure archiving of LAT diagnostic specific apids
	 
	

	cfg/vsc_cmd_apid.txt
	 VPI input file to configure archiving of VSC command specific apids
	 
	

	lci/latc_tkr.f
	LATC hardware configuration file for TKR calibrations.
	
	

	lci/latc_ign.f
	LATC ignore map
	
	

	lci/latc_lcm.f
	LATC master file
	
	

	lci/lci_tkr_dac63.f
	LCI TKR calibration configuration file with a fixed DAC injection of 63 and a threshold scan
	
	

	10k_event00
	FES data file. This file will not be specially crafted to represent TKR calibration data. Rather, it will simply allow the FES to simulate the required number of triggers.
	
	


6.3  Test Output Files

The following table identifies the log files, data dump files, and other output files generated by the test scripts executed during this qualification test. The “Output File Attachment Number” column identifies which attachment to this document is the hardcopy record of this script output data. The “Path to Attachment” column specifies where in the test repository an electronic copy of the hardcopy attachment has been saved.

	Output File
	Description of Output File
	Output File Attachment Number
	Path to Attachment

	VPI_apid_0x3c5_mmddyy_hhmmss.lsf
	LCI science data telemetry output file
	
	


6.4  Test Preparation

After the hardware and software setup has been validated, steps may be required to place the hardware and FSW in an operational mode in which the qualification test can be performed or otherwise complete preparations for the test to begin.

The Test Engineer carries out the following procedure to prepare for qualification tested and records completion of the test preparation steps in the space provided.

	Step No.
	Description of Step
	Step Outcome

	1
	Confirm that the VSC is powered up.


	Complete/

Not Complete

	2
	Confirm that the FES is powered up.
	Complete/

Not Complete

	3
	Confirm that the SIU and the GASU are powered up:

The voltage indicator on the SIU/GASU Xantrex power supply should display 28 ± 0.2V. The SIU feed "POWER ON" switch on the regulated feeds bus protection unit (BPU) should be flipped "ON". The SIU voltage indicator on the BPU should display 28 ± 0.2V. The SIU current as shown by the BPU indicator should be above 0.4A The DAQ feed "POWER ON" switch on the BPU should be flipped "ON". The DAQ voltage indicator on the BPU should display 28 ± 0.2V.


	Complete/

Not Complete


The Test Engineer and Quality Assurance Engineer verify that all test preparation steps are complete.

__________________   
_____________ ________________   ______________
Date

Time

Test Engineer

QAE


6.5  Test Procedure

After the test setup has been validated, and test preparation steps are complete, the Test Engineer proceeds with the qualification test procedure itself, as described below, and records the outcome of each step during test execution. The outcome of each step is either “Complete” or “Not Complete” (for steps which involve no analysis or verification); or, “Pass” or “Fail” (for steps involved in verifying completion of test objectives and sub-objectives).

	Step No.
	Description of Step
	Step Outcome


	1
	Set up the FSW to autoboot from the lower bank of SIU:

setenv LTX_C_AUTOBOOT 1

setenv VPI_AUTOBOOT_LO 0xc000

setenv VPI_AUTOBOOT_HI 0xA800

At the unix terminal, run the script FECALB_003.py using LTX with the following command:

ltx run FECALB_003 --ips=lat-hobbit2,lat-elf21,lat-ent1,lat-elf23 --protocol=ethernet,xyplex,xyplex,xyplex


	Complete/

Not Complete

	2
	The script initializes the system shells, loads the VSC software and starts proxy interface.

Output files listed in section 6.3 are created in the test session directory and the full path and filename of the soft copy of these output files are recorded in section 6.3 of the procedure document.


	Complete/

Not Complete

	3
	Initialization of FSW and front end electronics is performed:

Telecommand LPBCRTOSEXEC is issued to advance SIU through secondary boot, after which loading and initialization of FSW is performed

Telecommand LIMMAINFEEDON is sent to initialize the LCB, PDU, and GASU. 

Telecommand LIMPOWERON with parameters set to enable power to all instrument subsystems.

All LATC and LCI configuration files are uploaded to SIU ram


	N/A

	4
	FES initialization script is executed to configure and start the simulation.  LCICALIBRATE telecommand is sent using run ID 0x12341 and configuration files latc_tkr.f and lci_tkr_dac63.f.  This run is configured to emit 500 event samples at a 5 threshold points using a single DAC setting of 63.  The script waits for completion of the charge injection run, and the FES simulation is terminated.


	N/A

	5
	FES initialization script is executed to configure and start the simulation.  LCICALIBRATE telecommand is sent using run ID 0x12342 and configuration files latc_tkr.f and lci_tkr_dac63.f.  This run is configured to emit 500 event samples at a 5 threshold points using a single DAC setting of 63.  The script waits for completion of the charge injection run, and the FES simulation is terminated.


	N/A

	6
	The script closes all active systems:

Load shed is executed to disable power to all LAT electronics and reset SIU to primary boot mode.  

VSC reboot is executed.

Shell interfaces to all systems are closed.


	N/A

	7
	Press enter in terminal to end test.  This step will initiate execution of post-test analysis script.


	Complete/

Not Complete


Initial to confirm.

__________________   
_____________ ________________   ______________
Date

Time

Test Engineer

QAE


6.6  Test Analysis

The analysis for each of the sub-objectives is conducted by the main analysis script FECALB_003_analyze.py.  “Pass” or “Fail” is specified for steps involved in verifying completion of test objectives and sub-objectives.

	Step No.
	Description of Step
	Step Outcome


	1
	At the conclusion of the test run, the analysis script FECALB_003_analyze.py is autonomously executed.  Science data output file VPI_apid_0x3c5_mmddyy_hhmmss.lsf is parsed using DFI for each LCI datagram.  Each datagram is in turn parsed by the LDF iterator which extracts event data at the contributor level.

Review the terminal output of the analysis script for:

Analyzing test output


	Complete/

Not Complete

	2
	Datagrams from runs 0x12341 and 0x12342 are checked for events consistent with 5 threshold settings at the fixed DAC of 63.  Successful decoding of event datagrams verifies structure and content necessary to support trigger efficiency analysis.

Review the terminal output of the analysis script for:

TKR DAC 63 threshold scan points validated: PASS

TKR DAC 63 threshold scan samples validated: PASS

TKR DAC 63 parameters validated: PASS

TKR DAC 63 measurements validated: PASS


	Pass/Fail


7.  TEST POST-CONDITIONS AND OVERALL OUTCOME

7.1  Test Post-Conditions

The following post-conditions are analyzed and verified by the test script: 

	No.
	Post-Condition
	Post-Condition Met?
(Yes/No)

	1
	SIU is in LAT flight software is in BOOT mode as reported by the presence of boot telemetry packets 0x200

Session terminal log displays 

LAT LPBCBOOTSTAT 2 PBC_SW_MODE_COMMAND

SIU LPBCBOOTTYPE 5 COMMAND

	


The Test Engineer and Quality Assurance Engineer verify that all test post-conditions are met.

__________________   
_____________ ________________   ______________
Date

Time

Test Engineer

QAE


7.2  Overall Outcome of FECALB_003

Based on the analysis of the test results, the overall outcome of Test FECALB_003 is as follows:

□ Passed - all of the expected outcomes for the test were confirmed

□ Failed - one or more of the test outcomes were not confirmed

__________________  
________________   ______________

Date

    Test Engineer

QAE


8.  CERTIFICATION

I certify that the information obtained under this test procedure is as represented and the information recorded in this document is complete and correct. Any deviations from test procedures described herein are identified in Appendix A.

_________________

__________________________
________________________

Date



Test Engineer (Print Name)

Test Engineer (Signature)

I certify that the information obtained through execution of this test procedure is as represented and the information recorded in this document is complete and correct. Execution of the test, storage of the results, and verification of outcomes were carried out in accordance with quality standards defined in the GLAST Quality Manual (LAT-MD-00091).

_________________
___________________________
_____________________________

Date


Software QA Engineer (Print Name)
Software QA Engineer (Sign)


I certify that the information obtained under this test procedure is as represented and the information recorded in this document is complete and correct. The test procedure, as designed and executed, does indeed verify that the FSW functionality under test satisfies the corresponding requirements from the Flight Software Specification – Level III.

_________________

________________________
__________________________

Date



FSW Manager (Print Name)

FSW Manager (Signature)

Appendix A: Deviations from THE Qualification Test Procedure

This section details any deviations from the hardware configuration, software configuration, or test procedure followed during the execution of the test or tests described in this Qualification Test Procedure document. All deviations from the approved procedure are agreed to by the Test Engineer and the Software Quality Engineer during the test execution session. All deviations must be reported during the Post Qualification Test Review, where their impact on the test results will be evaluated.

Hardware Deviations

Describe any deviations from the hardware configuration defined in Section 6.1. Name the hardware that was modified and describe the modifications. If hardware is replaced during execution of the test, name the replaced hardware, the manufacturer, and list an identification number (e.g., GLAT ID number).

________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Software Deviations

Describe any changes made to the software configuration under test or the software configuration used to support test execution, as defined in Section 6.2. Give version numbers of all FSW packages and test packages that were modified. Describe how the contents of the modified software load were verified. Describe these deviations for each test that was modified.

________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Procedural Deviations

Specify any deviations from the test procedure for the test being executed. If this document contains more than one test procedure, list the procedure by number (e.g., “MEMMGT_003”). List by number the steps modified or skipped. Provide a numbered sequence listing any added steps. Describe these deviations for each test that was modified.

________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Hard copies of this document are for REFERENCE ONLY and should not be considered the latest revision.


