LAT-TD-07132-02    LAT FSW Qualification Test Procedure, FSWINI_001

        23 of 23

	[image: image1.jpg]



	Document #
	Date effective

	
	LAT-TD-07132-02
	11/10/05

	
	Author(s)
	Supersedes

	
	Igor Kouzmenko
	LAT-TD-07132-01

	
	
	

	
	
	

	
	Subsystem/Office

	
	Electronics & DAQ Subsystem

	Document Title

	LAT FSW Qualification Test Procedure: 
FSWINI_001: FSW Initialization -- SIU Primary Boot


Change History Log
	Revision
	Effective Date
	Description of Changes

	01
	August 16, 2005
	Original

	02
	11/10/05
	Reordered the sequence of steps in Test Procedure section (6.4) according to the order of test execution steps. 

Updated section 7.1


Table of Contents
3Table of Contents


51.
Scope


51.1
Test Suite


51.2
Test ID


61.3
Requirement(s) Tested


72.
DEFINITIONS AND ACRONYMS


72.1
Definitions


72.2
Acronyms


83.
REFERENCES


83.1
Applicable Documents


94.
REQUIREMENTS


94.1
Test Data, Equipment and Software


94.2
Quality Assurance


94.3
Safety


104.4
Warnings, Cautions, and Notes


104.5
General Instructions


115.
SETUP


115.1
Hardware Setup


125.2
Software Setup


145.3
Setup Validation


156.
TEST PROCEDURE FOR FSWINI_001


156.1
Test Objective


166.2
Test Input Files


166.3
Test Preparation


176.4
Test Procedure


197.
TEST POST-CONDITIONS AND OVERALL OUTCOME


197.1
Test Post-Conditions


197.2
Overall Outcome of FSWINI_001


208.
CERTIFICATION


21Appendix A: Deviations from THE Qualification Test Procedure





1. Scope

This document describes the procedure followed for qualification testing of the LAT FSW. This Qualification Test Procedure document describes one of the qualification tests executed to verify compliance with the requirements defined in the “Flight Software Specification – Level III” (LAT-SS-00399).
1.1 Test Suite
FSWINI
The Test Suite is a logical grouping of a set of tests based on common functionalities and can be executed as a suite of tests in some order when the pre-conditions and post-conditions for each test within the suite have been met successfully. 

This test procedure FSWINI_001 belongs in the Test Suite FSWINI under the FST Project.

1.2 Test ID
FSWINI_001
Note: Test FSWINI_001 contains one main script, FSWINI_001, which can be run when the SIU is unpowered.
The following telecommands and telemetry are used in this test:
Telecommands
	APID
	FC
	Cmd. Packet (L)
	Description (L)

	0x644
	0
	LMEMDUMPMEM
	Memory Data Dump


 Telemetry

	APID
	Tlm. Packet (L)
	Description (L)

	0x200
	LBTHKP
	Boot housekeeping telemetry

	0x314
	LMEMSIUDATA
	SIU Memory Dump Data


1.3 Requirements Tested

The Qualification Test Procedure described herein is performed to verify that the FSW satisfies the following requirement(s), quoted from the Flight Software Specification – Level III:

	Requirement Number
	Requirement Name
	Requirement
	Level of Requirements Verification in This Test

	5.3.1.1
	Power-up
	FSW shall perform a primary boot in response to SIU or EPU power-up.
	Partial

(FSWINI_001 verifies the power-up requirement on the SIU. FSWINI_005 verifies this requirement on the EPUs)

	5.3.1.2
	Boot from SUROM
	FSW shall perform a primary boot from RAD750 SUROM.
	Partial

(FSWINI_001 verifies the boot from SUROM requirement on the SIU. FSWINI_005 verifies this requirement on the EPUs)

	5.3.1.2.3
	RAM Initialization
	Upon power-up or hardware reset, the primary boot software shall initialize the RAM required to execute the FSW.
	Partial
(FSWINI_001 verifies the RAM initialization requirement on the SIU. FSWINI_005 verifies this requirement on the EPUs)

	5.3.1.3
	Boot with Watchdog Safeguard

	FSW shall disable the watchdog for at least 5 minutes to ensure the hardware watchdog does not activate during the primary boot.
	Partial 
(FSWINI_001 verifies the watchdog safeguard requirement on the SIU. FSWINI_005 verifies this requirement on the EPUs)


If the requirement(s) quoted above cite external documents (e.g., “…Further details are provided in [11]”), consult LAT-SS-00399 for the list of citations.
2. DEFINITIONS AND ACRONYMS
The following terms, abbreviations, and acronyms are used in this document:

2.1 Definitions

	Hz
	Hertz, unit of frequency 

	V
	Volt

	W
	Watt

	s, sec
	seconds

	
	

	
	


2.2 Acronyms

	CAL
	Calorimeter

	EGSE
	Electrical Ground Support Equipment

	GASU
	Global trigger Anti-collision Spacecraft Unit

	PTR
	Post Test Review

	SIB
	Storage and Interface Board

	TEM
	Tower Electronics Module

	TKR
	Tracker

	TPS
	Tower Power Supply

	TRR
	Test Readiness Review

	QAE
	Quality Assurance Engineer

	TE
	Test Engineer

	VSC
	Virtual Spacecraft

	
	


3. REFERENCES

The list below provides documents that are to be used as references for this procedure:
3.1 Applicable Documents

	Document Number
	Description

	SPECIFICATIONS
	

	LAT-SS-00399
	LAT Flight Software Level III Specification

	1196 EI-S46310-000
	GLAST 1553 Bus Protocol Interface Control

	LAT-TD-02659
	LAT Flight Software Telecommand and Telemetry Formats

	LAT-TD-0561
	The Virtual Spacecraft (VSC)

	
	

	PROCEDURES
	

	LAT-TD-07136
	FSWINI_005: FSW Initialization – EPU Primary Boot

	
	

	PLANS
	

	LAT-MD-00039
	Performance Assurance Implementation Plan

	LAT-MD-00078
	GLAST LAT System Safety Program Plan

	LAT-MD-00104 
	LAT FSW Management Plan

	LAT-MD-00404
	LAT Contamination Control Plan

	LAT-MD-00408
	LAT Program Instrument Performance Verification Plan

	LAT-SS-00296
	T & DF Test Plan

	LAT-TD-00297
	LAT Electronics Test Plan

	LAT-TD-00786
	LAT Flight Software Test Plan

	
	

	
	

	DRAWINGS
	

	N/A
	

	
	

	OTHER
	

	LAT-MD-00091
	GLAST Quality Manual

	LAT-MD-00471
	Control of Nonconforming Product

	LAT-MD-00472
	Corrective and Preventative Action

	
	BAE "RAD750 Board Appendix to the Hardware User's Manual" 

	
	

	
	


4. REQUIREMENTS

This section lists the requirements that shall be followed during the LAT FSW Qualification Testing process.

The Performance Assurance Implementation Plan, LAT-MD-00039, shall be utilized to ensure that the products produced by the GLAST LAT project intended for design qualification, flight and critical ground support equipment usage meet the required levels of quality and functionality for their intended purposes.

The LAT Program Instrument Performance Verification Plan, LAT-MD-00408, shall be utilized to address the testing to be performed at the unit/subsystem and instrument level for flight qualification, proto-flight and acceptance testing phases. Also included are the necessary processes/procedures and systems assurance activities.

4.1 Test Data, Equipment and Software

This procedure shall follow the requirements found in the Control of Nonconforming Product, LAT-MD-00471. This document establishes the method to identify and control nonconforming product developed by the LAT project team.

4.2 Quality Assurance

This procedure shall follow the requirements found in the Corrective and Preventative Action document, LAT-MD-00472 and the GLAST Quality Manual, LAT-MD-00091.

The Corrective and Preventative Action document establishes the method to be used to initiate, implement, evaluate and record corrective and preventive actions. The GLAST Quality Manual defines the methods implemented by the GLAST LAT project to ensure consistent quality of all processes for procurement, design, development and production of flight hardware, flight software and all associated ground support equipment interfacing with flight hardware and software.

4.3 Safety

This procedure shall follow the requirements found in the GLAST LAT System Safety Program Plan, LAT-MD-00078. This document defines all phases of the LAT program including: design, development, fabrication, handling, transportation, storage, test, assembly and operation.
WARNING: When high voltages are present extreme care should be exercised.

4.4 Warnings, Cautions, and Notes

The following SAFETY ALERTS are intended to create awareness of the potential safety hazards and the steps that must be taken to avoid accidents. These same alerts are used throughout this document to identify specific hazards that may endanger personnel and/or equipment.

Identification of every conceivable hazardous situation is impossible. Therefore, all personnel have the responsibility to diligently exercise safe practices whenever exposed to this equipment.

WARNING: Indicates a potential hazardous situation which, if not avoided, could result in death or injury.

CAUTION: Indicates a potential hazardous situation which, if not avoided, could result in damage to equipment.

Note: Indicates a notification of information that is important, but not hazard related.

4.5 General Instructions

This qualification test procedure shall be conducted on a formal basis to its latest approved and released version. The designated Software QAE shall be notified 24 hours prior to the start of this procedure. Software QAE may monitor the execution of all or part of this procedure should they elect to do so.

The Test Engineer conducting this test shall read this document in its entirety and resolve any apparent ambiguities before beginning the procedures described herein.

Deviations from the procedures described in this document and breaks in hardware or software configuration can only be initiated by the Test Engineer, must be approved by QA, and must be documented in Appendix A. 

Any nonconformance/defect/anomaly is to be reported in JIRA. Refer to the LAT Flight Software Test Plan LAT-TD-00786 for guidance. Do not alter or break configuration if a failure occurs. Notify Software Quality Assurance. 

All success conditions for a test must be met for the test to pass.
5. SETUP
This section describes the hardware and software configuration used for the qualification test described later in this document. Any break from configuration or deviation from a particular procedure must be authorized by the Quality Assurance Engineer and documented in Appendix A.

5.1 Hardware Setup

The list below indicates the equipment that is used to execute the tests described in this document.
	Hardware Unit
	Manufacturer
	GLAT Number (and Hardware Sub-Units by GLAT Number)
	Firmware Version (where applicable)

	Virtual Spacecraft (VSC) 
	SLAC
	
	

	Spacecraft Interface Unit (SIU)
	SLAC
	
	

	1553 cables and couplers
	SLAC
	
	

	Unix or Linux Host
	SLAC
	
	

	Power supply for SIU
	SLAC
	
	


The Figure below depicts the Testbed on which this qualification test is performed. The particular hardware units utilized in this test are itemized by GLAT number and firmware version in the preceding table.

[image: image2.png]From EBM

(GASU)
LVDS Science Heater Switch Environmental
Data Interface Monitor (LEDs) Simulator Boards
vsc
Host
1-PPS
(LTX proxy 1-PPS (GPS —
interface to 1
i or CPU clock) —
test scripts, (x2) PDU (x2)
test outputs) - -
1553 Environmental
mv2304 CPU|S 1553 J« swidhes {10 Registers
Ethernet [sumr 1553|
O Power Switches
Discrete pios| Mo [S[tce
| discretexs |
Ethernet Lines Board
Board *
GASU (x2)
Terminal VSC back-end VSC front-end
interface interface
I N
EPU (x3)
CRU
RAD750
SIB e '
< )
FES
Terminal
Iy
Ethernet
FES
TEM
Conmrs TEM (x16) TEM
(8 PC Boards)|
Front End Sim. Boards:
5 TKR: 1/twr = 16 boards
> (B cAL 1twr= 16 boaras
L,| fes |2 | AcD:2icomer =8 boards
Controller g
{0 Bcard) |ACD FES Board
1 Output to AEM
B | =
Clock Signal
ACD Controllers
(2 PC Boards) &@Ag —

n

0 VSC
LVDS Science
Interface




Figure 1. FSW Testbed
5.2 Software Setup

The software required to prepare for and execute the tests described in this Qualification Test Procedure document is itemized in this section.

5.2.1 Test Tools

The following table specifies the test executive used to run this qualification test, and identifies the other software tools used to support the execution of the test. The “Software Version Number” column identifies the version number of the test tool being used; alternatively, this column identifies the hardcopy attachment to this document that records the version of the tool being used (e.g., “Attachment 1”). The “Path to Attachment” column identifies the directory in which the electronic copy of any hardcopy attachment is saved (if applicable).
	Software
	Description of Software
	Software Version Number
(or Specify Attachment Number)
	Path to Attachment
(If Applicable)

	LTX
	LAT Test Executive
	
	

	FMX
	File Management Extra
	
	

	VSC
	Virtual Spacecraft system software 
	
	

	VPI
	VSC Python/Proxy Interface 
	
	


5.2.2 Test Scripts
The following table identifies the test scripts that are run to execute this qualification test. The “Script Version Number” column identifies which version of the script is being used; alternatively, this column identifies the hardcopy attachment to this document that records the version of the script being used (e.g., “Attachment 1”). The “Path to Attachment” column identifies the directory in which the electronic copy of any hardcopy attachment is saved (if applicable).
	Test Script
	Description of Test Script
	Script Version Number
(or Specify Attachment Number)
	Path to Attachment
(If Applicable)

	FSWINI_001.py
	The main LTX qualification test execution script for this test. This script must be run before the SIU RAD750 crate has powered up. The main part of this script may be useful to verify the pre-conditions for some tests that need to be conducted in the Boot Mode only. 
	
	

	STK
	This package contains the  utilities
	
	

	FSWINI_001.xml
	LTX test description file for test FSWINI_001.
	
	


5.2.3 Flight Software

This qualification test is performed on a complete, integrated Candidate Release of FSW. All FSW libraries under test are final Flight Unit Candidate versions. The test described in this document is designed to evaluate the particular FSW packages and constituents listed in the following table; for clarity, only those constituents that are the focus of the test are listed below. 
	FSW Package
	Constituent(s)

	PBC
	

	
	


The Candidate Release into which these FSW constituents are integrated is identified in the following table. The “Candidate Release Build Number” column specifies the build number of the FSW Candidate Release under test (e.g., “B0-0-1”). The “Candidate Release Attachment Number” column identifies which hardcopy attachment to this document confirms the name and version number of the full set of FSW packages and constituents used in the test (e.g., “Attachment 2”). The “Path to Attachment” column specifies where in the test repository an electronic copy of the hardcopy attachment has been saved. 
	Candidate Release Build Number
	Candidate Release Attachment Number
	Path to Attachment

	
	
	


5.3 Setup Validation

5.3.1 Hardware Validation

The following signatures confirm that the Test Engineer and Quality Assurance Engineer have verified the GLAT numbers, firmware version numbers, and the proper connection of all hardware listed in the table in Section 5.1.
__________________   
_____________ ________________   ______________
Date

Time

Test Engineer

QAE

5.3.2 Software Validation

The Test Engineer performs the following procedure to validate the software setup for this qualification test and records completion of the setup validation steps in the space provided.
	Step No.
	Description of Step
	Step Outcome

	1
	Record the version numbers of all test tools used to perform this qualification test in the table in Section 5.2.1
	Complete/
Not Complete

	2
	Record the version numbers of all test scripts used to perform this qualification test in the table in Section 5.2.2
	Complete/
Not Complete

	3
	Record the version numbers of the FSW constituents and the Candidate Release on which this test is performed in Section 5.2.3.
	Complete/
Not Complete

	4
	Record the version numbers of all input files used to perform this qualification test in the table in Section 6.2
	Complete/
Not Complete


The following signatures confirm that, using the procedure described in the previous table, the Test Engineer and Quality Assurance Engineer have verified that all versions of test support software, test scripts, input files, and FSW constituents match those identified in Sections 5.2 and 6.2.

__________________   
_____________ ________________   ______________
Date

Time

Test Engineer

QAE


6. TEST PROCEDURE FOR FSWINI_001 
6.1 Test Objective

This test verifies that SIU FSW performs a primary boot from RAD750 SUROM in response to SIU power-up. It also confirms that on power-up, the FSW initializes the RAM required to execute FSW applications and disables the watchdog for at least 5 minutes.
This Test Objective is broken down into the following Test Sub-Objectives.
	Number
	Test Sub-Objective

	1
	Verify that at least one primary boot image is burned into SUROM on the SIU’s RAD750 board and confirm that Primary Boot start from SUROM. 

	2
	Confirm primary boot is complete after power up by verifying transmission of SIU boot housekeeping telemetry.

	3
	Verify that the primary boot software initializes the RAM required to execute the FSW.

	4
	Verify that FSW disables the watchdog for at least 5 minutes to ensure the hardware watchdog does not activate during the primary boot. 


6.2 Test Input Files

The following table identifies all auxiliary files (e.g., Front End Simulator data files, GLEAM data files) used as inputs to this qualification test. Note that not all qualification tests use input data of this type. The “Input File Version Number” column identifies the version number of the auxiliary file being used; alternatively, this column identifies the hardcopy attachment to this document that records the version of the file being used (e.g., “Attachment 1”). The “Path to Attachment” column specifies where in the test repository an electronic copy of the hardcopy attachment has been saved .
	Input File
	Description of Input File
	Input File Version Number (or Specify Attachment Number)
	Path to Attachment
(If Applicable)

	FSWINI_001.xml
	LTX Configuration File
	
	


6.3 Test Output Files

The following table identifies all files used as outputs to this qualification test. Note that not all qualification tests use output data of this type. The “Output File Version Number” column identifies the version number of the auxiliary file being used; alternatively, this column identifies the hardcopy attachment to this document that records the version of the file being used (e.g., “Attachment 1”). The “Path to Attachment” column specifies where in the test repository an electronic copy of the hardcopy attachment has been saved.

	Output File
	Description of Output File
	Output File Version Number (or Specify Attachment Number)
	Path to Attachment
(If Applicable)

	
	
	
	

	
	
	
	

	
	
	
	


6.4 Test Preparation

After the hardware and software setup has been validated, steps may be required to place the hardware and FSW in an operational mode in which the qualification test can be performed or otherwise complete preparations for the test to begin.

The Test Engineer carries out the following procedure to prepare for qualification testing and records completion of the test preparation steps in the space provided.
	Step No.
	Description of Step
	Step Outcome

	1
	Confirm that the VSC is powered up and press the RESET button.
	Complete/
Not Complete

	2
	Power on SIU and GASU

The voltage indicator on the SIU/GASU Xantrex power supply should display 28 ± 0.2V. The SIU feed "POWER ON" switch on the regulated feeds bus protection unit (BPU) should be flipped "ON". The SIU voltage indicator on the BPU should display 28 ± 0.2V. The SIU current as shown by the BPU indicator should be above 0.4A The DAQ feed "POWER ON" switch on the BPU should be flipped "ON". The DAQ voltage indicator on the BPU should display 28 ± 0.2V.
	Complete/
Not Complete


The Test Engineer and Quality Assurance Engineer verify that all test preparation steps are complete.
__________________   
_____________ ________________   ______________
Date

Time

Test Engineer

QAE


6.5 Test Procedure

After the test setup has been validated, and test preparation steps are complete, the Test Engineer proceeds with the qualification test procedure itself, as described below, and records the outcome of each step during test execution. The outcome of each step is either “Complete” or “Not Complete” (for steps which involve no analysis or verification); or, “Pass” or “Fail” (for steps involved in verifying completion of test objectives and sub-objectives).  
	Step No.
	Description of Step
	Step Outcome

	1
	Run the FSWINI_001 test script by typing the following command at the shell:

$ ltx run <FSWINI_001 [command line arguments]
Mark ‘Complete’ for the Step Outcome if the test is not aborted.

	Complete/
Not Complete

	2
	LTX automatically sends several output files to the nfs file space. 

Record the full nfs path of the soft copy of the output files in the table in section 6.3:

_______________________________________________________________________ 
Print the FSWINI_001_fqt file, retrieve it from the printer, label it, and attach it to this test procedure.
	Complete/
Not Complete

	3
	Review the FSWINI_001_fqt output file and verify that the following entry appears:

FSWINI_001(1): Passed

This output indicates that the SIU FSW performed a proper Primary Cold Boot , at the end of which the FSW remained in the primary boot shell and transmitted boot housekeeping telemetry (LBTHKP packets) with APID = 0x200. Verify header and payload of boot housekeeping telemetry packets.

This step completes test sub-objective 1.

	Pass/
Fail

	4
	Review the  FSWINI_001_fqt output file and verify that the following entry appears:

FSWINI_001(2): Passed

- To confirm that the PBC code continues to use the SUROM, send LMEMDUMPREG command with offset 0x8c. The value of this register is related to the CPU program counter, and should be in the SUROM range (0xfff00000 - 0xfff3ffff).
This step completes test sub-objective 2.
	Pass/
Fail

	5
	Review the  FSWINI_001_fqt output file and verify that the following entry appears:

FSWINI_001(3): Passed
The PBC initializes RAM in two steps.  First, the RAM that is tested right after reset is initialized as a side-effect of the testing.  For cold boot, this is the entire RAM from address 0x00000000 to 0x00bfffff.  Second, the application/reserved RAM from 0x00c00000 to 0x07ffffff is initialized to all zeroes at the beginning of secondary boot. 

There's no way to access the RAM before the first step.

Between the first step of RAM init and the second step when PBC is running:

- Send LMEMLOADMEM command  to the  RAM above address 00c00000 (16 words) and  0x07ffffe0 (8 words) with random values;

- Get uninitialized RAM using LMEMDUMPREG command ;

- Reset the RAD750 with the primary boot flags set to re-run the RAM test (using the LPBCRESET command);

- Look at RAM again after the PBC restarts. 
This output indicates that RAM initialization was performed correctly. After initialization, RAM word data should be all 0’s.

This step completes test sub-objective 3.
	Pass/
Fail

	6
	Review the  FSWINI_001_fqt output file and verify that the following entry appears:

FSWINI_001(4): Passed
Trigger WATCHDOG  by sending  MEMLOADMEM command to the address 0xBF880058 with value 0 which initiates WATCHDOG type of reset. Resultant Boot Housekeeping Telemetry is checked to verify that the WATCHDOG type of reboot occurred in the BOOTYPE field. Also the Hi scrub addresses and error fields in this Telemetry are initialized too.
This step completes test sub-objective 4.
	Pass/
Fail

	7
	Confirm that each of the above entries appears in the log file with a “Pass” indication.

Successful completion of this step verifies all test objectives and sub-objectives for this test.
	Pass/

Fail


Initial to confirm.
__________________   
_____________ ________________   ______________
Date

Time

Test Engineer

QAE


7. TEST POST-CONDITIONS AND OVERALL OUTCOME
7.1 Test Post-Conditions

The expected state of the FSW and/or Testbed at the conclusion of this qualification test is indicated in the following table.
	No.
	Post-Condition
	Post-Condition Met?
(Yes/No)

	1
	The voltage indicator on the SIU/GASU Xantrex power supply should display 28 ± 0.2V. The SIU voltage indicator on the BPU should display 28 ± 0.2V. The SIU current as shown by the BPU indicator should be above 0.4A The DAQ feed "POWER ON" switch on the BPU should be flipped "ON". The DAQ voltage indicator on the BPU should display 28 ± 0.2V.
	


The Test Engineer and Quality Assurance Engineer verify that all test post-conditions are met.

__________________   
_____________ ________________   ______________
Date

Time

Test Engineer

QAE


7.2 Overall Outcome of FSWINI_001
Based on the analysis of the test results, the overall outcome of Test FSWINI_001 is as follows:

□ Passed - all of the expected outcomes for the test were confirmed
□ Failed - one or more of the test outcomes were not confirmed
__________________  
________________   ______________

Date

    Test Engineer

QAE


8. CERTIFICATION
I certify that the information obtained under this test procedure is as represented and the information recorded in this document is complete and correct. Any deviations from test procedures described herein are identified in Appendix A.
_________________

__________________________
________________________
Date



Test Engineer (Print Name)

Test Engineer (Signature)
I certify that the information obtained through execution of this test procedure is as represented and the information recorded in this document is complete and correct. Execution of the test, storage of the results, and verification of outcomes were carried out in accordance with quality standards defined in the GLAST Quality Manual (LAT-MD-00091).

_________________
___________________________
_____________________________
Date


Software QA Engineer (Print Name)
Software QA Engineer (Sign)

I certify that the information obtained under this test procedure is as represented and the information recorded in this document is complete and correct. The test procedure, as designed and executed, does indeed verify that the FSW functionality under test satisfies the corresponding requirements from the Flight Software Specification – Level III.

_________________

________________________
__________________________
Date



FSW Manager (Print Name)

FSW Manager (Signature)
Appendix A: Deviations from THE Qualification Test Procedure
This section details any deviations from the hardware configuration, software configuration, or test procedure followed during the execution of the test or tests described in this Qualification Test Procedure document. All deviations from the approved procedure are agreed to by the Test Engineer and the Software Quality Engineer during the test execution session. All deviations must be reported during the Post Qualification Test Review, where their impact on the test results will be evaluated.

Hardware Deviations

Describe any deviations from the hardware configuration defined in Section 5.1. Name the hardware that was modified and describe the modifications. If hardware is replaced during execution of the test, name the replaced hardware, the manufacturer, and list an identification number (e.g., GLAT ID number).

______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Software Deviations

Describe any changes made to the software configuration under test or the software configuration used to support test execution, as defined in Section 5.2. Give version numbers of all FSW packages and test packages that were modified. Describe how the contents of the modified software load were verified. Describe these deviations for each test that was modified.
______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Procedural Deviations
Specify any deviations from the test procedure for the test being executed. If this document contains more than one test procedure, list the procedure by number (e.g., “MEMMGT_001”). List by number the steps modified or skipped. Provide a numbered sequence listing any added steps. Describe these deviations for each test that was modified.
______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________



































































































































































































































































































































































































































































Hard copies of this document are for REFERENCE ONLY and should not be considered the latest revision.

Hard copies of this document are for REFERENCE ONLY and should not be considered the latest revision. 


