Track3P Top-Level Commands

- TotalTime

- ParticlesTrajectories

- FieldScales

- NormalizedField

- Emitter

- Domain

- Material

- OutputImpacts

- SingleParticleTrajectory

- Postprocess

Note: Refer to acdtool command syntax for postprocessing capabilities.

Back to Index

TotalTime

TotalTime: 50

TotalTime sets the total simulation time in rf cycles. Default is 20 rf cycles.

Back to Index

ParticlesTrajectories

ParticlesTrajectories:

{

ParticleFile: partpath

Start: 0

Stop: 1000000

Skip: 1

}

ParticlesTrajectories specifies the particle parameters for particle output iles, which can be used to visualize particles’ trajectories. It is only used for

ScanToken (in FieldScales) is 0.

ParticleFile defines the particle file names. Default is partpathXXXX.

Start: Start time step for writing particle files. Default is 0.

Stop: End time step for writing particle files. Default is 1000000.

Skip: Skip time step for writing particle files. Default is 1 for no skip.

Back to Index

FieldScales

FieldScales:

{

Type: FieldGradient

ScanToken: 1

Minimum: 23.0e+6

Maximum: 25.0e+6

Interval: 1.0e+6

Scale: 23.00e+6

}

FieldScales specifies the parameters for the field levels where track3p simulations are carried out. The field level is represented in different types, and it can be single level or scanned.

Type defines the physical quantity that is used to define the field level. There are three types

- FieldGradient: The average field gradient along a specified path defined by the container NormalizedField [unit in V/m]

- InputPortPower: The power at the input port [unit in W]

- StorEnergy: The stored energy of the rf field [unit in J]

ScanToken: 0 or 1

0: A single field level set by Scale; files on particle tracking will be written.

1: Scan field level from Minimum to Maximum in intervals defined by Interval; no particle tracking files will be written.

Minimum: The minimum scanned field level [unit defined by Type]. For example, if FieldGradient is used, the unit is V/m. It is used when ScanToken is set to 1.

Maximum: The maximum scanned field level [unit defined by Type]. For example, if FieldGradient is used, the unit is V/m. It is used when ScanToken is set to 1.

Interval: The interval for scanned field levels [unit defined by Type]. For example, if FieldGradient is used, the unit is V/m. It is used when ScanToken is set to 1.

Scale:  The single field level  [unit defined by Type]. For example, if FieldGradient is used, the unit is V/m. It is used when ScanToken is set to 0.

Back to Index

NormalizedField

NormalizedField:

{

Beta: 1

StartPoint: 0.0 0.0 -0.057

EndPoint: 0.0 0.0 0.057

N: 300

}

NormalizedField specifies the coordinates of the start and end points of the straight path that is used to normalize the average field gradient. It is only used when Type in the container FieldScales is FieldGradient.

Beta: Beta parameter for field calculation, default = 1.0

StartPoint: x, y, z

The coordinates (x, y, z) of the start point of the path [units in m]

EndPoint: x, y, z

The coordinates (x, y, z) of the end point of the path [units in m]

N: Number of inner points in the gradient integral line. Default is 300.

Back to Index

Emitter

Emitter:

{

Type: 2

t0: 0

t1: 1

x0: -0.03

x1: 0.035

y0: 0.04

y1: 0.1

z0: -0.045

z1: 0.0

BoundaryID: 6

SkipTimeSteps: 1

N: 1000

WorkFunction: 4.2

Beta: 120

}

Emitter defines the locations and duration for the emission of initial particles (primary particles). Multiple Emitter can be defined on a single mesh surface.

Type: Defines how the particles are emitted and can be

- ElectronicSurface or 2: Initial particles are emitted from the mesh surface inside a bounding box. This is the default option.

- SolidElectronicSurface or 5: Initial particles are emitted from the mesh surfaces as well as the boundaries of different volumetric regions. This is used for multipacting simulation of rf windows which have different material regions.

- FieldEmission or 7: Primary particles are emitted following Fowler Nordheim Law. This is used for dark current simulation.

- ParticlesFile or 10: Load particles from a file for particle tracking.

  - ParticlesFile: particles.dat

  - Format of particle data:

num. electrons/ energy(ev) / position_x(m) / y(m) / z(m) / momentum_x / y / z

t0 and t1: Describe the start and end times in rf cycles when particles are emitted. Default it ranges from 0 to 1, but other values can be used.

x0, x1, y0, y1, z0 and z1: The bounding box for particle emission from the surface or volumetric boundaries [units in m]

BoundaryID: The boundary surface ID for particle emission

SkipTimeSteps: Emission skip time step. Default is 1 for no skip.

N:  Minimum number of electrons in an emitted macro particle. Particles will not be emitted if the number of electrons is less than this value. Only used for field emission

WorkFunction: Work function for field emission [unit in eV]. Default is 4.2 eV for copper. Only used for field emission

Beta: Beta value for field emission. Only used for field emission

Back to Index

Back to Index

Domain

Domain:

{

FieldDir: ./omega3p_results

ModeID: 0

FrequencyScanID: 0 // for S3P only

dt: 1

MaxImpacts: 50

LowEnergy: 20

HighEnergy: 5000

InitialEnergy: 2.0

Voltage: 0.0 0.0 0.0

Bins: 64

// For rf windows

SolidRegion: 2

VacuumRegion: 1

// With uniform external magnetic field

ExternalMagneticField: 0.0 0.0 4.0

// With external magnetic field map

MagneticFieldMap:

{

File: bfield2

Scaling: 0.0001

Units: 0.001

ZOffset:0.27

}

// With electrostatic field

ElectroStaticFieldMap: {

File: ./gun3p_results/OUTPUT/E/test-11.txt

Scaling: 1.0

}

SymmetryFactor: 2

SymmetryPlane: 5

}

Domain specifies field parameters for various types of simulation condition. This includes simulations with rf fields, external static field, or with rf windows.

FieldDir: The directory storing the omega3p or s3p results. Track3P will read in a mode file from the directory.

ModeID: Specifies the mode number calculated using omega3p or port mode ID for s3p imported into track3p. The default is the first eigenmode for Omega3P and the first port mode ID for S3P,  starting with the number 0.

FrequencyScanID: Eigensystem ID from frequency scan in S3P

dt: Parameter used for time step. Default dt = 1.0, timestep = dt/frequency/100, and default timestep = 1.0/frequency/100.

MaxImpacts: Maximum impact number, after which particles will die. Default is 50.

LowEnergy and HighEnergy: Define the impact energy range, beyond which no secondary particles are emitted when a primary particle hit a surface. The energy units are in eV.

InitialEnergy:  Initial energy of emitted particels [unit in eV]. Default is 2.0 eV.

Bins: Parameter used for localizing particles, with the maximum number of bins set in the longest direction among those in x, y, z and the other remaining two directions’ bins derived from it. Default is 64.

Voltage: External voltage for coax, with input format (a ra rb), where the units in a, ra, rb are [V, m, m] and voltage = a/log(rb/ra). Default in (0 0 0) for no external voltage.

SolidRegion: Non-vacuum region, whose block ID is set in Cubit. It is only used for simulation models with rf windows.

VacuumRegion: Vacuum region, whose block ID is set in Cubit. It is only used for simulation models with rf windows.

ExternalMagneticField: Bx, By, Bz

Uniform external magnetic field (Bx, By, Bz) [units in T]

MagneticFieldMap: External magnetic field map to be imported

- File: The name of the file containing the external magnetic field map

Track3p read field maps with cylindrical symmetry. Data iare read in from an ASCII "Poisson" file containing (Fr,Fz,Fphi) as a function of z and r positions. The field map is formatted either as follows with for 3 field components:

r_0 r_{Nr+1} Nr                 // from .. to .. number of intervals in r direction

z_0 z_{Nz+1} Nz               // from .. to .. number of intervals in z direction

Fr Fz Fphi (at z_0, r_0)   // field values at the corresponding points, there need to be (Nr+1)      * (Nz+1) lines starting from here

Fr Fz Fphi (at z_0, r_1)

...

Fr Fz Fphi (at z_0, r_{Nr+1})

Fr Fz Fphi (at z_1, r_0)

...

Fr Fz Fphi (at z_1, r_{Nr+1})

…

Fr Fz Fphi (at z_Nz, r_{Nr+1})

or, as follows for 2 field components:

r_0 r_{Nr+1} Nr

z_0 z_{Nz+1} Nz

Fr Fz (at z_0, r_0)

Fr Fz (at z_0, r_1)

...

Fr Fz (at z_0, r_{Nr+1})

Fr Fz (at z_1, r_0)

...

Fr Fz (at z_1, r_{Nr+1})

...

Fr Fz (at z_Nz, r_{Nr+1})

- Scaling: The field scale factor to multiply the fields in the original field map

- Units: The distance scale factor to convert original coordinates in the original field map to meters used in track3p

- ZOffset: The z-offset of the origin [unit in m] when importing the original field map into track3p simulation model

ElectroStaticFieldMap: External electrostatic field map to be imported

- File: The name of the file containing the external electrostatic field map

- Scaling: The field scale factor to multiply the fields in the original field map

SymmetryFactor:  symmetry plane factor. 2: half; 4: quarter; 1: full

SymmetryPlane:  symmetry plane boundary surface ID

Back to Index

Back to Index

Material

Material:{

Type: Primary

BoundarySurfaceID:  6

}

Material:{

Type: Secondary

BoundarySurfaceID: 3 4 6

}

Material:{

Type: Absorber

BoundarySurfaceID:  8 7

}

// For rf windows:

Material:{

Type: Primary

SolidBlockID:  2

}

Material:{

Type: Secondary

SolidBlockID:  1

}

Material:{

Type: Absorber

BoundarySurfaceID:  7 8

}

Material:{

Type: Secondary

BoundarySurfaceID: 6

Model: 2

SecondaryEmissionYield: sey.dat

MinimumNumElectrons: 1000

ElasticThreshold: 1000

}

Material specifies the characteristics of particles on a surface.

Type: Type of surface

- Primary: Particles are emitted.

- Secondary: Secondary particles are emitted when a particle hit the surface.

- Absorber: The particle is absorbed and lost when it hits the surface.

BoundarySurfaceID: Reference number of boundary surface, which is set in Cubit.

SolidBlockID: Reference number of the block ID for solid material, which is set in Cubit, It is only used for rf windows.

Model: defines different types of secondary emission. Default is 1.

1:	For multipacting simulation. When a particle hits a surface, one secondary particle will be emitted from the same location with angle normal to the surface.

2:	For dark current simulation. When a particle hits a surface, true secondary and reflected particles will be emitted with random angles.

3:	Furman-Pivi secondary emission model. When a particle hits a surface, multiple secondary particles will be emitted following Furman-Pivi model.

MinimumNumElectrons: Minimum number of electrons in each secondary macro particle. If the number of electrons is less than this number, no secondary particle is emitted. This parameter only applies to Model 2.

ElasticThreshold: Minimum number of electrons in each secondary reflected-type macro particle. If the number of electrons is less than this number, no secondary reflected-type particle is emitted. This parameter only applies to Model 2.

SecondaryEmissionYield: Secondary emission yield data file formatted as:

Column 1: impact energy specified in eV.

Column 2:corresponding secondary emission yield coefficient

Back to Index

Back to Index

OutputImpacts

OutputImpacts: off

OutputImpacts specifies if particle impact information will be written.

on: Particle impact information will be dumped into files for detailed studies.

off: No particle impact information will be saved.

Back to Index

SingleParticleTrajectory

SingleParticleTrajectory: off

SingleParticleTrajectory specifies if all single particle trajectory will be written in separated files. Default is off.

on: Each single particle trajectory will be written in a file named “particleID”.

off: No single particle trajectory will be dumped.

Back to Index

Postprocess

Postprocess: {

Toggle: on

ResonantParticles: {

Token: on

InitialImpacts        :    4

EnergyRange         : 25 10000

PhaseTolerance    :   0.01

}

EnhancementCounter: {

Token: on

SEYFileName1: copper.txt

BoundarySurfaceID1: 6

SEYFileName2: niobium.txt

BoundarySurfaceID2: 3

SolidVolID: 0

MinimumEC: 0.01

}

}

Postprocess specifies the option for postprocessing of data from multipacting simulations.

Toggle: on or off

on: Turn on the postprocessing procedure.

off: Turn off the postprocessing procedure.

ResonantParticles: Define the parameters for tracking resonant particles.

- Token: on or off

  - on: Turn on the tracking of resonant particles.

  - off: Turn off the tracking of resonant particles.

- InitialImpacts: Particles with impact number greater than the number of initial impacts are considered. Default is 4.

- EnergyRange: E1, E2

The energy range between E1 and E2 to consider if a particle exhibits a resonant trajectory [units in eV]. When the impact energy of the particle is out of this range, it will be discarded from the list of resonant particles.

- PhaseTolerance:  Phase tolerance to determine resonant particles

EnhancementCounter: Specifies the parameters used to calculate the particle enhancement counter. The enhancement counter is defined as the total number of secondary electrons originated form a single primary electron during the number of rf cycles in a simulation. It is written as

where δi is the secondary emission yield (SEY) at the ith impact and its value is determined by the impact energy and the SEY curve. The parameters for setting the enhancement  counter are

- Token: on or off

on: Turn on the enhancement counter.

off: Turn off the enhancement counter.

- BoundarySurfaceID1: The boundary surface ID, which is set in Cubit, used for calculating the enhancement counter

- SEYFileName1: The name of the file containing the SEY for the corresponding material on BoundarySurfaceID1. The first and second columns in the file are the impact energy and the number of emitted particles due to a single particle impact, respectively.

- BoundarySurfaceID2: The boundary surface ID, which is set in Cubit, used for calculating the enhancement counter

- SEYFileName2: The name of the file containing the SEY for the corresponding material on BoundarySurfaceID2. The first and second columns in the file are the impact energy and the number of emitted particles due to a single particle impact, respectively.

- SolidVolID: Window volume ID, which is set in Cubit

- MinimumEC: Minimum enhancement counter, less than which no data will not be written to file. Default is 10.

Back to Index